An Evaluation of Multi-Agent Reinforcement Learning for Dynamic Bus Scheduling Under Non-Ideal Conditions

Progress Report — Advances Since the Revised Manuscript

Group B3 · University of Santo Tomas · CapMetro Rapid Route 801 (Direction 6, Austin TX) · 4 September 2026

# 1  Overview

This report documents the work completed since the revised manuscript, from the processing of the CapMetro Automatic Passenger Counter (APC) records through to a preliminary comparison of holding controllers under ideal and non-ideal conditions. It is written to be read alongside the manuscript and does not alter it; the submitted document remains the governing text, and every procedure reported here is consistent with the methodology it sets out. The account proceeds in the order the pipeline executes: data processing, corridor construction and calibration, the disturbance layer, the derivation of the control stops, the multi-agent reinforcement-learning (MARL) formulation and its training harness, and the baseline evaluation. Each section states what was done, grounds the procedure in the literature the manuscript draws on, gives the governing equations, shows the operative code, and presents the resulting figure or table. Full source listings appear in the appendix.

The scope of the empirical study is a single high-frequency corridor: the twenty-six stops of Route 801 in direction 6. The dynamic-holding problem on such a corridor is the setting studied by the cooperative and multi-agent reinforcement-learning holding literature (Rodriguez et al., 2023; Wang & Sun, 2020, 2023; Chen et al., 2016), and the abstract, empirically parameterised corridor adopted here — as distinct from a street-level road-network reconstruction — is the modelling convention in that body of work.

# 2  Data Processing

The empirical basis is the CapMetro APC dataset for July-December 2021. A single streamed pass over the raw asset reduces roughly nine million stop-events to the 229,421 events belonging to Route 801 in direction 6. Six rules define the subset: the record must be on route 801; the operating route must equal the scheduled route, which removes mid-service reassignments; the two import-error flags must be clear; the stop identifier must be valid; and the direction code must be 6. The subset is written to disk and a SHA-256 checksum is recorded so that the derived inputs are reproducible and auditable, a property requested in review.

Thirteen of the forty-seven APC fields are retained, each for a defined purpose: two identify the route and screen for reassignment (route_id, current_route_id); two are data-quality gates (import_error, import_trip_error); two locate the event (bs_id, direction_code_id); and the remainder carry stop order (actual_sequence), demand (ons, offs), dwell (dwell_time), and segment time and distance (rev_seconds, rev_distance), with the timestamp retained for ordering. Per-stop parameters are then reduced from the subset: the median is used for dwell, running time, and distance, which is robust to the long-tailed layover and outlier records a mean would distort, and the mean is used for boardings and alightings, which are the demand rates the disturbance generators sample. The segment running time is defined as rev_seconds - dwell_time so that the originating stop's dwell is not counted twice.

Excerpt — scripts/extract_sim_inputs.py: the six-rule direction-6 filter and the per-stop reduction.

for ch in pd.read_csv(RAW, usecols=need, dtype=str, chunksize=1_000_000, na_filter=False):
    n_raw += len(ch)
    m = ((ch["route_id"]=="801") & (ch["route_id"]==ch["current_route_id"]) &
         (ch["import_error"]=="0") & (ch["import_trip_error"]=="0") &
         (ch["bs_id"]!="0") & (ch["direction_code_id"]=="6"))
    parts.append(ch[m])
df = pd.concat(parts, ignore_index=True)
print(f"raw {n_raw:,} -> dir-6 clean {len(df):,} (expect 229,421)")
df.to_csv("data/route_801_direction_6_clean.csv", index=False)      # cache the subset
for c in ["actual_sequence","ons","offs","dwell_time","rev_seconds","rev_distance"]:
    df[c] = pd.to_numeric(df[c], errors="coerce")
df["run_seconds"] = (df["rev_seconds"] - df["dwell_time"]).clip(lower=0)   # segment running time
stops = (df.groupby("bs_id")
           .agg(seq=("actual_sequence","median"), mean_boardings=("ons","mean"),
                mean_alightings=("offs","mean"), dwell_s=("dwell_time","median"),
                run_s=("run_seconds","median"), dist_mi=("rev_distance","median"))  # NOTE: rev_distance is MILES
           .sort_values("seq").round(2))

The result is sim_inputs/stops.csv, twenty-six rows of per-stop dwell, running time, distance, and demand that parameterise the simulation. No passenger-arrival times are used, consistent with the manuscript's treatment of arrivals as unobserved; demand enters only as a rate.

# 3  Corridor Construction and Calibration

The corridor is parameterised directly from the APC record rather than reconstructed as a street-level microsimulation. This choice is deliberate and grounded: the APC coordinates are stop-event points, not continuous trajectories, so a calibrated road network cannot be justified from the data, and empirically parameterised abstract corridors are standard in the bus-holding reinforcement-learning literature (Rodriguez et al., 2023; Wang & Sun, 2020). The simulator is SUMO (Lopez et al., 2018). Stops are placed at their cumulative along-route distances and one edge joins each consecutive pair; a single bus is run and each edge speed is solved so that simulated segment travel time matches the empirical median.

Because buses are injected at the observed service frequency, hourly counts match by construction, so the GEH statistic is applied to segment travel time as a closeness measure and the root-mean-square percentage error (RMSPE) is treated as the binding travel-time criterion. The GEH acceptance threshold of five, met on at least eighty-five percent of segments, follows established microsimulation-calibration guidance (Federal Highway Administration, 2019; UK Highways Agency, 1996); weather- and travel-time-aware calibration of this kind is the subject of recent forecasting work (Patil et al., 2025).

GEH = √( 2(M − C)² / (M + C) )        (1)

M, C — simulated and observed segment value; accepted when GEH < 5 on > 85% of segments.

RMSPE = √( (1/n) Σᵢ ((Mᵢ − Cᵢ) / Cᵢ)² ) × 100%        (2)

binding travel-time error over the n = 25 segments.

vᵢ = Lᵢ / rᵢ ,   then   vᵢ ← vᵢ · (Mᵢ / Cᵢ)        (3)

edge speed; Lᵢ along-route distance, rᵢ empirical running time; iterated until the criterion is met.

Calibration converges to a travel-time RMSPE of 0.75 percent with GEH below five on every one of the twenty-five segments — well inside the acceptance region. The iteration that solves the edge speeds is shown below, followed by the calibration outcome.

Excerpt — scripts/calibrate_corridor.py: the GEH/RMSPE evaluation and the multiplicative speed refinement.

def main():
    speeds = [dist[i] / target[i] for i in range(len(dist))] + [10.0]
    for it in range(1, 13):
        M = build_and_run(speeds); k = len(M); C = target[:k]
        geh = np.sqrt(2 * (M - C) ** 2 / (M + C))
        rmspe = np.sqrt(np.mean(((M - C) / C) ** 2)) * 100
        ok = np.mean(geh < 5) * 100
        print(f"iter {it}: <5:{ok:.0f}% RMSPE={rmspe:.2f}% GEHmax={geh.max():.2f}")
        if ok >= 85 and rmspe < 2.0:
            print(f"calibration criterion met (RMSPE {rmspe:.2f}%, GEH<5 on {ok:.0f}%) -> sumo/corridor.* is calibrated")
            print("final segment times (s):", [round(v) for v in M])
            os.makedirs("results", exist_ok=True)
            with open("results/calibration.csv", "w") as fh:
                fh.write("segment,observed_s,simulated_s,geh,pct_err\n")
                for i in range(k):
                    fh.write(f"{CORRIDOR[i]}-{CORRIDOR[i+1]},{C[i]:.0f},{M[i]:.0f},{geh[i]:.2f},{(M[i]-C[i])/C[i]*100:+.1f}\n")
            print(f"wrote results/calibration.csv (RMSPE {rmspe:.2f}%)"); return
        for i in range(k): speeds[i] *= M[i] / C[i]

Figure 1. Calibrated versus observed segment travel times across the corridor; points lie on the identity line (RMSPE 0.75%, all segments GEH < 5).

| Segment | Obs (s) | Sim (s) | GEH | % err |
| --- | --- | --- | --- | --- |
| 5280-5857 | 370 | 369 | 0.05 | -0.3 |
| 5857-5858 | 156 | 156 | 0.04 | +0.3 |
| 5858-4540 | 119 | 120 | 0.09 | +0.8 |
| 4540-467 | 163 | 164 | 0.08 | +0.6 |
| 467-5859 | 106 | 109 | 0.29 | +2.8 |
| 5859-5606 | 193 | 193 | 0.00 | +0.0 |
| 5606-5861 | 192 | 192 | 0.00 | +0.0 |
| 5861-6360 | 151 | 150 | 0.08 | -0.7 |
| 6360-484 | 151 | 151 | 0.00 | +0.0 |
| 484-5405 | 153 | 154 | 0.08 | +0.7 |
| 5405-5357 | 150 | 150 | 0.00 | +0.0 |
| 5357-5863 | 134 | 134 | 0.00 | +0.0 |
| 5863-497 | 126 | 125 | 0.09 | -0.8 |
| 497-5866 | 182 | 180 | 0.15 | -1.1 |
| 5866-2738 | 139 | 138 | 0.08 | -0.7 |
| 2738-2611 | 91 | 91 | 0.00 | +0.0 |
| 2611-5867 | 103 | 103 | 0.00 | +0.0 |
| 5867-6372 | 278 | 278 | 0.00 | +0.0 |
| 6372-4029 | 222 | 221 | 0.07 | -0.5 |
| 4029-4046 | 184 | 184 | 0.00 | +0.0 |
| 4046-5870 | 146 | 146 | 0.00 | +0.0 |
| 5870-5553 | 244 | 242 | 0.13 | -0.8 |
| 5553-6356 | 214 | 212 | 0.14 | -0.9 |
| 6356-5871 | 176 | 176 | 0.00 | +0.0 |
| 5871-5872 | 179 | 179 | 0.00 | +0.0 |

Table 1. Per-segment calibration for the twenty-six-stop corridor (25 segments). Maximum GEH 0.29; travel-time RMSPE 0.75%.

# 4  Disturbance Layer

Five disturbance families are modelled — demand variability (D), a localised demand surge (S), traffic-speed variation (T), weather (W), and a vehicle breakdown (B) — and are combined through an activation matrix used in the evaluation. The single most consequential modelling decision is that dwell time depends on the number of waiting passengers. Without that coupling a demand surge would change nothing and headway irregularity would have no physical driver; with it, a delayed bus accumulates more passengers, dwells longer, and falls further behind, reproducing the Newell-Potts bunching instability by which small perturbations amplify along a corridor (Daganzo, 2009; Yang et al., 2024). Service time is a fixed component plus a per-passenger boarding term, capped, and multiplied by a pre-drawn lognormal noise field.

Excerpt — envs/corridor_sim.py: demand-responsive service time and the per-stop control decision.

try: nwait = traci.busstop.getPersonCount(s)
                    except traci.TraCIException: nwait = 0
                    dserv = min(MAX_SERV, FIXED_DWELL + BOARD_S * nwait) * DWN[bi, i]
                    hold = 0.0
                    if i in control_stops and bi > 0:
                        hf = t - barr[(bi-1, i)] if (bi-1, i) in barr else H0
                        hb = (barr[(bi, i-1)] - barr[(bi+1, i-1)]) if ((bi+1, i-1) in barr and (bi, i-1) in barr) else H0
                        try: load = traci.vehicle.getPersonNumber(v)
                        except traci.TraCIException: load = 0
                        w = float(WF[bi, i]) if W else 1.0                    # weather intensity (upcoming segment)
                        obs = dict(hf=hf, hb=hb, load=load, queue=nwait, idx=i, n=ns, H0=H0, cap=CAP_PAX,
                                   bus=bi, w=w, b=1.0 if bk_triggered else 0.0)
                        hold, _skip = decide(obs)
                        hold = float(max(0.0, min(hold, CAP * H0)))
                    tb = TBREAK if (B and bi == bk_idx and i == bks) else 0.0
                    if tb > 0: bk_triggered = True                            # incident now on the corridor
                    target[v] = max(FIXED_DWELL, dserv) + hold + tb

All stochastic factors are pre-generated as fixed fields indexed by bus and stop, so that two controllers evaluated at the same seed face an identical disturbance realisation; a sequential draw was rejected because the order in which controllers consume it depends on their holding decisions, which would break the pairing. The weather factor is heavy-tailed lognormal with unit mean, clipped to a physical range so that a bus cannot be brought to a standstill. It is labelled synthetic throughout: the manuscript reserves the empirical NOAA precipitation join as the primary weather exposure, and that join remains future work.

dᵢ = min( d_max , d₀ + β·nᵢ ) · εᵢ ,   εᵢ ~ LogNormal(0, σ_d)        (4)

demand-responsive service time; d₀ = 6 s, β = 4 s/passenger, d_max = 90 s, σ_d = 0.25; nᵢ waiting passengers.

F_W ~ clip( LogNormal(−σ²/2, σ) , 0.5 , 3 ) ,   σ² = ln(1 + η²)        (5)

weather factor; E[F_W] = 1, CV = η (default 0.8); applied segment speed vᵢ / F_W.

F_T ~ Uniform(0.8, 1.2)        (6)

traffic-speed factor; applied segment speed vᵢ · F_T.

# 5  Control-Stop Selection

The stops at which holding is permitted are derived from the four criteria in the manuscript applied to the demand and through-volume profile, not chosen for convenience. The origin terminal is included; the onset of each above-mean-demand segment is a candidate; stops carrying high through-volume are excluded so that holding does not penalise large numbers of in-transit passengers; and where adjacent stops both qualify, only the upstream one is kept. The procedure yields five control stops — bus_ids 5280, 5857, 5859, 5867, and 4046 — used identically by every controller so that any performance difference is attributable to the control logic and not to a difference in the number of intervention points.

Excerpt — envs/corridor_sim.py: the derived control-stop set and the heuristic control laws.

# Control stops derived from the four §3.2.2 criteria on the demand / through-volume profile:
# origin terminal (idx 0); onset of each above-mean-demand segment (idx 1,5,9,17,20); drop high
# through-volume (idx 9 removed); upstream-only for adjacent hubs (none). -> 5 stops.
CONTROL_STOPS = [0, 1, 5, 17, 20]                    # bs_ids 5280, 5857, 5859, 5867, 4046
 
os.makedirs("sumo", exist_ok=True)
if not os.path.exists("sumo/vtype.add.xml"):                         # guard: parallel workers don't race
    open("sumo/vtype.add.xml", "w").write('<additional><vType id="bus" vClass="bus" length="12" '
        'accel="1.2" decel="4.0" maxSpeed="30" personCapacity="60"/></additional>\n')
 
# ---- baseline decide functions (a controller is just decide: obs -> (hold_s, skip)) --------------
def fh_hold(hf): return 0.0 if hf >= H0 else float(min(H0 - hf, CAP * H0))
def eh_hold(hf, hb): return float(max(0.0, min(0.5 * (hb - hf), CAP * H0)))
def nc_decide(o): return 0.0, 0
def fh_decide(o): return fh_hold(o["hf"]), 0
def eh_decide(o): return eh_hold(o["hf"], o["hb"]), 0
BASELINES = {"NC": nc_decide, "FH": fh_decide, "EH": eh_decide}

Whether five stops confer enough control authority was measured rather than assumed. Restricting the heuristics to these five stops under ideal demand still reduces headway irregularity by roughly a third relative to no control, which establishes sufficiency; the full results follow in Section 7.

# 6  MARL Formulation and Training Harness

The learned controller is a parameter-shared Double Deep Q-Network (Double DQN) trained under centralised training with decentralised execution (CTDE). The discrete ten-action space makes a value-based method the natural choice (Mnih et al., 2015), and the double estimator is adopted over plain DQN because the heavy-tailed disturbance regime amplifies the action-value overestimation the double estimator is designed to remove (van Hasselt et al., 2016). One shared network serves every bus, which makes the policy invariant to fleet size (Gupta et al., 2017; Christianos et al., 2021); each bus contributes its own local transition and per-agent reward to a shared replay buffer, with no joint state or team reward. This mirrors the cooperative and multi-agent holding formulations of Wang & Sun (2020, 2023) and Rodriguez et al. (2023). Because control is event-driven, each bus's transition is assembled across consecutive decisions and an action's reward is realised at that bus's next decision — a semi-Markov formulation (Bradtke & Duff, 1995). The environment is built on the standard Gymnasium and PettingZoo interfaces (Towers et al., 2024; Terry et al., 2021).

Each agent observes the seven-vector of Table 3.6 — control-stop index, forward headway, estimated backward headway, onboard load, waiting queue, a weather flag, and a breakdown flag — and selects one of ten actions formed from five holding fractions and a binary skip. The reward is a three-term penalty on headway irregularity, passenger waiting, and skip degeneracy; the weighting coefficients are the Expected Output 2.1 tuning deliverable and are treated as configurable rather than fixed. Passenger waiting is weighted by boardings, consistent with the value-of-time basis for wait cost (Wardman, 2004).

R_i = E[ Σ₍ₖ≥₀₎ e^(−β t_k) · r_{i,t+k} ]        (7)

event-discounted return; t_k elapsed time to the k-th control event for bus i.

r = −( w₁·irr + w₂·wait + w₃·skip )        (8)

three-term penalty; w₁, w₂, w₃ the EO 2.1 coefficients (tuned).

y = r + γ (1 − done) · Q( s′ , argmaxₐ′ Q(s′, a′; θ) ; θ⁻ )        (9)

Double-DQN target; θ online network, θ⁻ target network; γ discount.

Excerpt — agents/ddqn.py: the parameter-shared Double-DQN update (online selects, target evaluates).

def learn(self):
        self.steps += 1
        if len(self.buf) < max(self.batch, self.warmup):
            return None
        o, a, r, o2, d = (x.to(self.device) for x in self.buf.sample(self.batch))
        q = self.q(o).gather(1, a.unsqueeze(1)).squeeze(1)
        with torch.no_grad():
            a2 = self.q(o2).argmax(1)                                   # online selects
            q2 = self.qt(o2).gather(1, a2.unsqueeze(1)).squeeze(1)      # target evaluates (double-Q)
            y = r + self.gamma * (1.0 - d) * q2
        loss = nn.functional.smooth_l1_loss(q, y)
        self.opt.zero_grad(); loss.backward()
        nn.utils.clip_grad_norm_(self.q.parameters(), 10.0)
        self.opt.step()
        if self.steps % self.target_every == 0:
            self.qt.load_state_dict(self.q.state_dict())
        return float(loss.item())

The harness is deliberately design-agnostic: the reward terms, the observation, and every hyperparameter are exposed as configuration so that alternatives can be compared without editing the model, and no design choice is committed until the evidence supports it. A short fail-fast gate guards the training budget — a brief run that must show the policy beating no control before a full budget is committed, so that a mis-specified reward or observation is caught cheaply. Under the gate the shared policy converges to the level of the fixed-holding heuristics on the headway-irregularity metric, confirming the apparatus learns.

Figure 2. Fail-fast gate: evaluation headway-CV of the shared policy over training, converging toward the fixed-holding baseline and below no control.

# 7  Baseline Evaluation

Three controllers are compared — No-Control (NC), Forward-Headway holding (FH), and two-way Even-Headway holding (EH) — over a matched-seed Monte-Carlo of thirty replications per cell, with ninety-five-percent confidence intervals obtained by bootstrap resampling (Efron & Tibshirani, 1993). Controllers act only at the five designated stops and are evaluated on identical disturbance realisations at each seed, so that differences are attributable to the control logic. The activation matrix comprises a Stage-A regime of demand and traffic (D+T), single-factor ablations that add the surge, weather, or breakdown, and a Stage-B regime combining all five factors. Passenger wait is reported as the expected wait under random arrivals given the realised headways, and cross-checked against SUMO's direct per-passenger wait; the two agree under mild conditions and diverge under heavy weather, where far-stop arrivals lag the injection window.

| Scenario | Ctrl | Headway CV [95% CI] | Travel s [95% CI] | Wait s [95% CI] | wdir |
| --- | --- | --- | --- | --- | --- |
| Stage A (D+T) | NC | 0.331 [0.307, 0.356] | 5043 [5029, 5057] | 166 [164, 169] | 167 |
|  | FH | 0.237 [0.224, 0.250] | 5134 [5119, 5150] | 160 [159, 162] | 162 |
|  | EH | 0.271 [0.257, 0.285] | 5089 [5075, 5103] | 162 [160, 164] | 163 |
| Ablation S (D+T+S) | NC | 0.364 [0.345, 0.384] | 5160 [5140, 5179] | 169 [166, 171] | 285 |
|  | FH | 0.304 [0.288, 0.319] | 5287 [5264, 5311] | 166 [164, 167] | 310 |
|  | EH | 0.316 [0.296, 0.336] | 5216 [5198, 5234] | 166 [164, 168] | 294 |
| Ablation W (D+T+W) | NC | 0.977 [0.918, 1.038] | 6454 [6306, 6615] | 296 [281, 312] | 475 |
|  | FH | 0.918 [0.863, 0.975] | 6626 [6467, 6798] | 290 [272, 311] | 499 |
|  | EH | 0.893 [0.813, 0.988] | 6437 [6271, 6613] | 280 [255, 313] | 446 |
| Ablation B (D+T+B) | NC | 0.442 [0.407, 0.478] | 5099 [5084, 5114] | 179 [175, 184] | 183 |
|  | FH | 0.365 [0.331, 0.402] | 5206 [5190, 5223] | 174 [170, 178] | 180 |
|  | EH | 0.376 [0.345, 0.408] | 5139 [5125, 5153] | 173 [170, 177] | 177 |
| Stage B (D+T+S+W+B) | NC | 0.941 [0.891, 0.992] | 6777 [6595, 6959] | 280 [262, 298] | 742 |
|  | FH | 0.929 [0.872, 0.988] | 6965 [6804, 7140] | 294 [275, 313] | 828 |
|  | EH | 0.929 [0.867, 0.989] | 6852 [6661, 7044] | 289 [268, 311] | 776 |

Table 2. Monte-Carlo results (N = 30 matched seeds per cell). Wait is the headway-model expected wait; wdir is the direct SUMO per-passenger cross-check.

The paired percentage changes relative to no control isolate each controller's effect on the same seeds. Under the Stage-A regime both controllers reduce headway irregularity significantly — Forward-Headway by twenty-eight percent and Even-Headway by eighteen percent, with confidence intervals excluding zero. The reduction persists under the surge and breakdown ablations. Under heavy weather, however, the effect is small and its interval includes zero, and under the combined Stage-B regime the controllers fall to no-control levels: fixed holding does not withstand the compounded disturbance. This degradation is the central finding motivating a learned controller, and it is the gap the MARL policy is intended to close.

| Scenario | FH Δ CV % [95% CI] | FH Δ wait % | EH Δ CV % [95% CI] | EH Δ wait % |
| --- | --- | --- | --- | --- |
| Stage A (D+T) | -28% [-35, -21] | -4% | -18% [-25, -11] | -3% |
| Ablation S (D+T+S) | -17% [-22, -11] | -2% | -13% [-19, -7] | -2% |
| Ablation W (D+T+W) | -6% [-12, +1] | -2% | -9% [-17, +1] | -5% |
| Ablation B (D+T+B) | -17% [-23, -12] | -3% | -15% [-19, -10] | -3% |
| Stage B (D+T+S+W+B) | -1% [-6, +3] | +5% | -1% [-7, +5] | +4% |

Table 3. Paired percentage change versus No-Control (negative = controller better). Intervals that exclude zero denote significance.

Figure 3. Headway coefficient of variation by scenario and controller (N = 30; bars are 95% bootstrap CIs). Control gains shrink as disturbances compound.

Figure 4. Time-space (Marey) trajectories over the corridor; converging lines indicate bunching, which holding acts to re-space.

Figure 5. Headway CV against weather intensity: controller advantage narrows as weather severity rises, motivating an adaptive policy.

# 8  Objectives Status and Remaining Work

The simulation environment and its calibration (Specific Objective 1) are complete. The baseline evaluation under ideal and non-ideal conditions (Specific Objective 3, for the heuristic controllers) is complete, with the finding that fixed holding degrades to no-control levels under weather and the combined regime. The reinforcement-learning apparatus (Specific Objective 2) is built and validated through the fail-fast gate. The remaining work is, in order: the reward-coefficient study conducted through the gate; the full training run under domain randomisation over the activation matrix; the evaluation of the trained policy as a fourth controller on the same cells, completing the ideal and non-ideal assessment; and the empirical NOAA precipitation join to complement the synthetic weather regime. The visual route overlay from OpenStreetMap and a street-level microsimulation remain presentation- and future-work items and do not affect the reported results.

# References

Bradtke, S. J., & Duff, M. O. (1995). Reinforcement learning methods for continuous-time Markov decision problems. Advances in Neural Information Processing Systems, 7, 393-400.

Chen, W., Zhou, K., & Chen, C. (2016). Real-time bus holding control on a transit corridor based on multi-agent reinforcement learning. 19th IEEE Intl. Conf. on Intelligent Transportation Systems (ITSC), 100-106.

Christianos, F., Papoudakis, G., Rahman, A., & Albrecht, S. V. (2021). Scaling multi-agent reinforcement learning with selective parameter sharing. Proc. 38th Intl. Conf. on Machine Learning (ICML), PMLR 139, 1989-1998.

Daganzo, C. F. (2009). A headway-based approach to eliminate bus bunching: Systematic analysis and comparisons. Transportation Research Part B, 43(10), 913-921.

Efron, B., & Tibshirani, R. J. (1993). An Introduction to the Bootstrap. Chapman & Hall/CRC.

Federal Highway Administration. (2019). Traffic Analysis Toolbox Volume III: Guidelines for Applying Traffic Microsimulation Modeling Software (FHWA-HOP-18-036). U.S. Dept. of Transportation.

Gupta, J. K., Egorov, M., & Kochenderfer, M. J. (2017). Cooperative multi-agent control using deep reinforcement learning. AAMAS 2017 Workshops, LNCS 10642, 66-83.

Liu, D., Xiao, F., Luo, J., & Yang, F. (2023). Deep reinforcement learning-based holding control for bus bunching under stochastic travel time and demand. Sustainability, 15(14), 10947.

Lopez, P. A., Behrisch, M., Bieker-Walz, L., Erdmann, J., Floetteroed, Y.-P., Hilbrich, R., Luecken, L., Rummel, J., Wagner, P., & Wiessner, E. (2018). Microscopic traffic simulation using SUMO. 21st IEEE ITSC, 2575-2582.

Mnih, V., Kavukcuoglu, K., Silver, D., et al. (2015). Human-level control through deep reinforcement learning. Nature, 518(7540), 529-533.

Patil, M., Ahmed, Q., & Midlam-Mohler, S. (2025). Travel time and weather-aware traffic forecasting in a conformal graph neural network framework. IEEE Trans. on Intelligent Transportation Systems, 26(12), 21734-21744.

Rodriguez, J., Koutsopoulos, H. N., Wang, S., & Zhao, J. (2023). Cooperative bus holding and stop-skipping: A deep reinforcement learning framework. Transportation Research Part C, 155, 104308.

Terry, J. K., Black, B., Grammel, N., et al. (2021). PettingZoo: Gym for multi-agent reinforcement learning. Advances in Neural Information Processing Systems, 34.

Towers, M., Kwiatkowski, A., Terry, J. K., et al. (2024). Gymnasium: A standard interface for reinforcement learning environments. arXiv:2407.17032.

UK Highways Agency. (1996). Design Manual for Roads and Bridges, Vol. 12, Sec. 2: Traffic Appraisal in Urban Areas. The Stationery Office, London.

van Hasselt, H., Guez, A., & Silver, D. (2016). Deep reinforcement learning with double Q-learning. Proc. 30th AAAI Conf. on Artificial Intelligence, 2094-2100.

Wang, J., & Sun, L. (2020). Dynamic holding control to avoid bus bunching: A multi-agent deep reinforcement learning framework. Transportation Research Part C, 116, 102661.

Wang, J., & Sun, L. (2023). Robust dynamic bus control: A distributional multi-agent reinforcement learning approach. IEEE Trans. on Intelligent Transportation Systems, 24(4), 4075-4088.

Wardman, M. (2004). Public transport values of time. Transport Policy, 11(4), 363-377.

Yang, Y., Cheng, J., & Liu, Y. (2024). An overview of solutions to the bus bunching problem in urban bus systems. Frontiers of Engineering Management.

# Appendix  —  Full Code Listings

The complete implementation, read directly from the working tree. Each listing is preceded by its path and one-line role.

## scripts/extract_sim_inputs.py

Raw APC CSV → direction-6 clean subset (229,421 rows) → per-stop simulation inputs.

"""Stream the 3.5 GB raw APC CSV -> dir-6 clean subset (229,421 rows) -> sim_inputs/stops.csv.
Reproduces the pipeline's six cleaning filters. Point values only (no distributions)."""
import pandas as pd, os
RAW = r"C:\Users\jared\Desktop\THESIS\APC_Raw_July_2021_-_December_2021_20260824.csv"  # <-- set path
OUT = "sim_inputs"; os.makedirs(OUT, exist_ok=True); os.makedirs("data", exist_ok=True)
need = ["route_id","current_route_id","import_error","import_trip_error","bs_id","direction_code_id",
        "actual_sequence","ons","offs","dwell_time","rev_seconds","rev_distance","transit_date_time"]
parts, n_raw = [], 0
for ch in pd.read_csv(RAW, usecols=need, dtype=str, chunksize=1_000_000, na_filter=False):
    n_raw += len(ch)
    m = ((ch["route_id"]=="801") & (ch["route_id"]==ch["current_route_id"]) &
         (ch["import_error"]=="0") & (ch["import_trip_error"]=="0") &
         (ch["bs_id"]!="0") & (ch["direction_code_id"]=="6"))
    parts.append(ch[m])
df = pd.concat(parts, ignore_index=True)
print(f"raw {n_raw:,} -> dir-6 clean {len(df):,} (expect 229,421)")
df.to_csv("data/route_801_direction_6_clean.csv", index=False)      # cache the subset
for c in ["actual_sequence","ons","offs","dwell_time","rev_seconds","rev_distance"]:
    df[c] = pd.to_numeric(df[c], errors="coerce")
df["run_seconds"] = (df["rev_seconds"] - df["dwell_time"]).clip(lower=0)   # segment running time
stops = (df.groupby("bs_id")
           .agg(seq=("actual_sequence","median"), mean_boardings=("ons","mean"),
                mean_alightings=("offs","mean"), dwell_s=("dwell_time","median"),
                run_s=("run_seconds","median"), dist_mi=("rev_distance","median"))  # NOTE: rev_distance is MILES
           .sort_values("seq").round(2))
stops.to_csv(f"{OUT}/stops.csv"); print("wrote sim_inputs/stops.csv", len(stops), "stops")

## scripts/calibrate_corridor.py

Builds the corridor net and solves edge speeds until GEH<5 / RMSPE target.

"""First SUMO calibration pass for the reduced corridor.
 
Builds a SUMO net from real stop coordinates (geometry), sets dwells from real medians,
and iterates edge speeds until simulated segment travel times match the empirical run_s
targets. Acceptance: GEH < 5 on > 85% of segments (methods §3.2.3), reported with RMSPE.
 
NOTE ON GEH: methods define GEH on hourly bus VOLUME; when buses are injected at the
observed frequency, volume matches by construction. Here GEH is applied to segment TRAVEL
TIMES as a closeness statistic, paired with RMSPE (which is the binding travel-time metric).
"""
import os, sys, subprocess, math
import numpy as np, pandas as pd
if "SUMO_HOME" in os.environ:
    sys.path.insert(0, os.path.join(os.environ["SUMO_HOME"], "tools"))
import traci
from sumolib import checkBinary
 
NC, SUMO = checkBinary("netconvert"), checkBinary("sumo")
CORRIDOR = [l.strip() for l in open("corridor.txt") if l.strip()]   # full 26-stop dir-6 corridor
os.makedirs("sumo", exist_ok=True)
 
coords = pd.read_csv("sim_inputs/stop_coordinates.csv").set_index("bs_id")
si = pd.read_csv("sim_inputs/stops.csv").set_index("bs_id")
ids = [int(x) for x in CORRIDOR]
c, d = coords.loc[ids], si.loc[ids]
lat0, lon0 = c["mean_lat"].mean(), c["mean_lon"].mean()
X = (c["mean_lon"] - lon0) * math.cos(math.radians(lat0)) * 111320
Y = (c["mean_lat"] - lat0) * 110540
xy = list(zip(X.values, Y.values))
dist = [math.hypot(xy[i+1][0]-xy[i][0], xy[i+1][1]-xy[i][1]) for i in range(len(ids)-1)]
target = d["run_s"].values[:len(dist)]
dwell = d["dwell_s"].values
 
 
def build_and_run(speeds):
    n = len(CORRIDOR)
    xs = [0.0]
    for L in dist: xs.append(xs[-1] + L)
    xs.append(xs[-1] + 500)
    open("sumo/corridor.nod.xml", "w").write("<nodes>\n" + "".join(f'  <node id="n{i}" x="{x:.1f}" y="0"/>\n' for i, x in enumerate(xs)) + "</nodes>\n")
    open("sumo/corridor.edg.xml", "w").write("<edges>\n" + "".join(f'  <edge id="e{i}" from="n{i}" to="n{i+1}" numLanes="1" speed="{speeds[i]:.2f}"/>\n' for i in range(n)) + "</edges>\n")
    subprocess.run([NC, "--node-files=sumo/corridor.nod.xml", "--edge-files=sumo/corridor.edg.xml", "--output-file=sumo/corridor.net.xml"], check=True, capture_output=True)
    open("sumo/stops.add.xml", "w").write("<additional>\n" + "".join(f'  <busStop id="{CORRIDOR[i]}" lane="e{i}_0" startPos="5" endPos="25"/>\n' for i in range(n)) + "</additional>\n")
    stopxml = "".join(f'    <stop busStop="{CORRIDOR[i]}" duration="{dwell[i]:.0f}"/>\n' for i in range(n))
    open("sumo/corridor.rou.xml", "w").write(
        '<routes>\n  <vType id="bus" vClass="bus" length="12" accel="1.2" decel="4.0" maxSpeed="30"/>\n'
        f'  <route id="r" edges="{" ".join(f"e{i}" for i in range(n))}"/>\n'
        f'  <vehicle id="b0" type="bus" route="r" depart="0">\n{stopxml}  </vehicle>\n</routes>\n')
    open("sumo/corridor.sumocfg", "w").write(
        '<configuration>\n <input>\n  <net-file value="corridor.net.xml"/>\n  <route-files value="corridor.rou.xml"/>\n'
        '  <additional-files value="stops.add.xml"/>\n </input>\n <time><begin value="0"/><end value="9000"/></time>\n</configuration>\n')
    traci.start([SUMO, "-c", "sumo/corridor.sumocfg", "--no-warnings", "true", "--no-step-log", "true"])
    arr, dep, prev, t = [], [], False, 0
    while traci.simulation.getMinExpectedNumber() > 0 and t < 9000:
        traci.simulationStep(); t = traci.simulation.getTime()
        s = traci.vehicle.isStopped("b0") if "b0" in traci.vehicle.getIDList() else False
        if s and not prev: arr.append(t)
        if not s and prev: dep.append(t)
        prev = s
    traci.close()
    return np.array([arr[i+1] - dep[i] for i in range(min(len(arr)-1, len(dep), len(dist)))])
 
 
def main():
    speeds = [dist[i] / target[i] for i in range(len(dist))] + [10.0]
    for it in range(1, 13):
        M = build_and_run(speeds); k = len(M); C = target[:k]
        geh = np.sqrt(2 * (M - C) ** 2 / (M + C))
        rmspe = np.sqrt(np.mean(((M - C) / C) ** 2)) * 100
        ok = np.mean(geh < 5) * 100
        print(f"iter {it}: <5:{ok:.0f}% RMSPE={rmspe:.2f}% GEHmax={geh.max():.2f}")
        if ok >= 85 and rmspe < 2.0:
            print(f"calibration criterion met (RMSPE {rmspe:.2f}%, GEH<5 on {ok:.0f}%) -> sumo/corridor.* is calibrated")
            print("final segment times (s):", [round(v) for v in M])
            os.makedirs("results", exist_ok=True)
            with open("results/calibration.csv", "w") as fh:
                fh.write("segment,observed_s,simulated_s,geh,pct_err\n")
                for i in range(k):
                    fh.write(f"{CORRIDOR[i]}-{CORRIDOR[i+1]},{C[i]:.0f},{M[i]:.0f},{geh[i]:.2f},{(M[i]-C[i])/C[i]*100:+.1f}\n")
            print(f"wrote results/calibration.csv (RMSPE {rmspe:.2f}%)"); return
        for i in range(k): speeds[i] *= M[i] / C[i]
 
 
if __name__ == "__main__":
    main()

## envs/corridor_sim.py

Reusable simulation core: one loop any controller (baseline or MARL) drives via decide(obs).

"""Reusable corridor simulation core — one loop that ANY controller drives.
 
`simulate(decide, ...)` runs the calibrated 26-stop corridor with the demand-responsive dwell and the
D/S/T/W/B disturbance generators, and calls `decide(obs) -> (hold_seconds, skip)` whenever a bus reaches
a CONTROL stop. Baselines (NC/FH/EH) and the MARL policy are just different `decide` functions, so they
all run on an identical environment — apples-to-apples by construction.
 
obs (dict) at a control stop:
    hf, hb  forward / backward headway (s)      load   onboard passengers
    queue   passengers waiting at the stop       idx    stop index (0..n-1)
    n       number of stops                      H0     scheduled headway (s), cap  bus capacity
decide returns (hold_seconds, skip_bool). hold is clipped to [0, 0.4*H0]; skip is accepted but only
takes effect if the caller enabled skipping (SKIP_ENABLED) — the fail-fast gate runs holding-only.
 
Run from the repo root (needs the calibrated sumo/ net + sim_inputs + corridor.txt).
"""
import os, sys, math, numpy as np, pandas as pd, xml.etree.ElementTree as ET
if "SUMO_HOME" in os.environ:
    sys.path.insert(0, os.path.join(os.environ["SUMO_HOME"], "tools"))
import traci, sumolib
from sumolib import checkBinary
 
STOPS = [l.strip() for l in open("corridor.txt") if l.strip()]
EDGES = [f"e{i}" for i in range(len(STOPS))]
co = pd.read_csv("sim_inputs/stop_coordinates.csv").set_index("bs_id").loc[[int(s) for s in STOPS]]
d  = pd.read_csv("sim_inputs/stops.csv").set_index("bs_id").loc[[int(s) for s in STOPS]]
lat0, lon0 = co["mean_lat"].mean(), co["mean_lon"].mean()
Xs = (co["mean_lon"] - lon0) * math.cos(math.radians(lat0)) * 111320
Ys = (co["mean_lat"] - lat0) * 110540
DIST = [math.hypot(Xs.values[i+1]-Xs.values[i], Ys.values[i+1]-Ys.values[i]) for i in range(len(STOPS)-1)]
SEGV = [DIST[i] / d["run_s"].values[i] for i in range(len(DIST))]
BASE = {STOPS[i]: max(8.0, float(d["dwell_s"].values[i]))        for i in range(len(STOPS))}
DEM  = {STOPS[i]: max(0.0, float(d["mean_boardings"].values[i])) for i in range(len(STOPS))}
H0, NBUS, CVD, CAP, BIG, TBREAK, ETA = 300.0, 12, 0.25, 0.4, 600.0, 400.0, 0.8
FIXED_DWELL, BOARD_S, MAX_SERV, CAP_PAX = 6.0, 4.0, 90.0, 60
RUN = [float(d["run_s"].values[i]) for i in range(len(STOPS))]
CUM = [0.0]
for i in range(1, len(STOPS)):
    CUM.append(CUM[-1] + RUN[i-1] + BASE[STOPS[i-1]])
SURGE_I = len(STOPS) // 3
INTERIOR = list(range(1, len(STOPS) - 1))            # stops eligible to be control points
# Control stops derived from the four §3.2.2 criteria on the demand / through-volume profile:
# origin terminal (idx 0); onset of each above-mean-demand segment (idx 1,5,9,17,20); drop high
# through-volume (idx 9 removed); upstream-only for adjacent hubs (none). -> 5 stops.
CONTROL_STOPS = [0, 1, 5, 17, 20]                    # bs_ids 5280, 5857, 5859, 5867, 4046
 
os.makedirs("sumo", exist_ok=True)
if not os.path.exists("sumo/vtype.add.xml"):                         # guard: parallel workers don't race
    open("sumo/vtype.add.xml", "w").write('<additional><vType id="bus" vClass="bus" length="12" '
        'accel="1.2" decel="4.0" maxSpeed="30" personCapacity="60"/></additional>\n')
 
# ---- baseline decide functions (a controller is just decide: obs -> (hold_s, skip)) --------------
def fh_hold(hf): return 0.0 if hf >= H0 else float(min(H0 - hf, CAP * H0))
def eh_hold(hf, hb): return float(max(0.0, min(0.5 * (hb - hf), CAP * H0)))
def nc_decide(o): return 0.0, 0
def fh_decide(o): return fh_hold(o["hf"]), 0
def eh_decide(o): return eh_hold(o["hf"], o["hb"]), 0
BASELINES = {"NC": nc_decide, "FH": fh_decide, "EH": eh_decide}
 
 
def _make_persons(S, path):
    p = ["<additional>"]
    for i in range(len(STOPS) - 1):
        K = int(round(NBUS * DEM[STOPS[i]]))
        if K > 0:
            b, e = int(CUM[i]), int(CUM[i] + H0 * (NBUS - 1))
            p.append(f'<personFlow id="f{i}" begin="{b}" end="{e}" number="{K}">'
                     f'<stop busStop="{STOPS[i]}" duration="1"/><ride busStop="{STOPS[-1]}" lines="801"/></personFlow>')
    if S:
        sb = int(CUM[SURGE_I] + H0)
        p.append(f'<personFlow id="surge" begin="{sb}" end="{sb+900}" number="120">'
                 f'<stop busStop="{STOPS[SURGE_I]}" duration="1"/><ride busStop="{STOPS[-1]}" lines="801"/></personFlow>')
    open(path, "w").write("\n".join(p + ["</additional>"]))
 
 
def simulate(decide, seed=0, D=True, S=False, T=False, W=False, B=False, control_stops=None,
             eta=None, trace=False):
    """Drive the corridor with controller `decide`. Returns metrics dict (headway_cv, travel_s, wait_s).
    eta overrides the weather CV (for severity sweeps); trace=True also returns per-bus stop arrivals."""
    control_stops = set(INTERIOR if control_stops is None else control_stops)
    rng = np.random.default_rng(1000 + seed)
    ns = len(STOPS)
    _eta = ETA if eta is None else eta                                   # weather intensity (sweepable)
    DWN = rng.lognormal(0.0, CVD, size=(NBUS, ns))                       # dwell noise (pre-drawn field)
    s2 = math.log(1 + _eta * _eta)
    WF = np.clip(rng.lognormal(-0.5 * s2, math.sqrt(s2), size=(NBUS, ns)), 0.5, 3.0)   # weather
    TF = rng.uniform(0.8, 1.2, size=(NBUS, ns))                          # traffic
    bk_idx = int(rng.integers(2, NBUS - 1)); bks = int(rng.integers(1, ns - 1)) if B else -1
    port = sumolib.miscutils.getFreeSocketPort(); started = False
    tri = f"sumo/tri_{port}.xml"; persons = f"sumo/persons_{port}.xml"    # per-call, parallel-safe
    _make_persons(S, persons)
    try:
        traci.start([checkBinary("sumo"), "-n", "sumo/corridor.net.xml",
                     "-a", f"sumo/vtype.add.xml,sumo/stops.add.xml,{persons}",
                     "--tripinfo-output", tri, "--no-warnings", "true", "--no-step-log", "true",
                     "--seed", str(seed), "--step-length", "1", "-e", "36000"], port=port); started = True
        traci.route.add("corr", EDGES)
        departs = {f"b{i}": i * H0 for i in range(NBUS)}
        added, toinit, idx, prev = set(), set(), {}, {}
        arr = {s: [] for s in STOPS}; tarr, target, resumed, entry, ca = {}, {}, set(), {}, {}
        barr = {}
        bk_triggered = False           # True once a breakdown is in effect (obs breakdown flag b)
        t = 0.0
        while t < H0 * NBUS + 30000 and (traci.simulation.getMinExpectedNumber() > 0 or len(added) < NBUS):
            for v, dep in departs.items():
                if v not in added and t >= dep:
                    traci.vehicle.add(v, "corr", typeID="bus", line="801"); added.add(v); toinit.add(v)
            traci.simulationStep(); t = traci.simulation.getTime(); live = set(traci.vehicle.getIDList())
            for v in list(toinit):
                if v in live:
                    idx[v], prev[v], entry[v] = 0, False, t
                    for k in range(ns):
                        try: traci.vehicle.setBusStop(v, STOPS[k], duration=BIG)
                        except traci.TraCIException: pass
                    toinit.discard(v)
            for v in list(added):
                if v not in live or v in toinit: continue
                st, i = traci.vehicle.isStopped(v), idx[v]
                if st and not prev[v] and i < ns:
                    s = STOPS[i]; arr[s].append(t); tarr[v] = t; bi = int(v[1:]); barr[(bi, i)] = t
                    if i == ns - 1: ca[v] = t
                    try: nwait = traci.busstop.getPersonCount(s)
                    except traci.TraCIException: nwait = 0
                    dserv = min(MAX_SERV, FIXED_DWELL + BOARD_S * nwait) * DWN[bi, i]
                    hold = 0.0
                    if i in control_stops and bi > 0:
                        hf = t - barr[(bi-1, i)] if (bi-1, i) in barr else H0
                        hb = (barr[(bi, i-1)] - barr[(bi+1, i-1)]) if ((bi+1, i-1) in barr and (bi, i-1) in barr) else H0
                        try: load = traci.vehicle.getPersonNumber(v)
                        except traci.TraCIException: load = 0
                        w = float(WF[bi, i]) if W else 1.0                    # weather intensity (upcoming segment)
                        obs = dict(hf=hf, hb=hb, load=load, queue=nwait, idx=i, n=ns, H0=H0, cap=CAP_PAX,
                                   bus=bi, w=w, b=1.0 if bk_triggered else 0.0)
                        hold, _skip = decide(obs)
                        hold = float(max(0.0, min(hold, CAP * H0)))
                    tb = TBREAK if (B and bi == bk_idx and i == bks) else 0.0
                    if tb > 0: bk_triggered = True                            # incident now on the corridor
                    target[v] = max(FIXED_DWELL, dserv) + hold + tb
                    try: traci.vehicle.setMaxSpeed(v, 30.0)
                    except traci.TraCIException: pass
                if st and v not in resumed and (t - tarr.get(v, t)) >= target.get(v, 0):
                    try: traci.vehicle.resume(v); resumed.add(v)
                    except traci.TraCIException: pass
                    if i < len(DIST):
                        f = 1.0
                        if W: f *= float(WF[bi, i])
                        if T: f /= float(TF[bi, i])
                        if f != 1.0:
                            try: traci.vehicle.setMaxSpeed(v, max(2.0, SEGV[i] / f))
                            except traci.TraCIException: pass
                if (not st) and prev[v] and i < ns: idx[v] = i + 1; resumed.discard(v)
                prev[v] = st
    finally:
        if started:
            try: traci.close()
            except Exception: pass
    cvs = [np.std(np.diff(sorted(arr[s]))) / np.mean(np.diff(sorted(arr[s]))) for s in STOPS[1:] if len(arr[s]) >= 3]
    tt  = [ca[v] - entry[v] for v in ca if v in entry]
    # wait_model: expected passenger wait under random arrivals given the *realized* bus headways,
    #   w = (E[H]/2)(1+CV^2), boardings-weighted — robust to demand-injection timing (primary).
    num, den = 0.0, 0.0
    for s in STOPS[1:]:
        h = np.diff(sorted(arr[s]))
        if len(h) >= 2 and DEM[s] > 0:
            Hb = h.mean(); cv = h.std() / Hb; num += DEM[s] * 0.5 * Hb * (1 + cv * cv); den += DEM[s]
    # wait_direct: SUMO's per-passenger recorded wait (cross-check; matches wait_model under mild
    #   conditions, inflates under heavy weather where far-stop arrivals lag the injection window).
    waits = []
    try:
        for pi in ET.parse(tri).getroot().findall("personinfo"):
            for r in pi.findall("ride"):
                if float(r.get("arrival", "-1")) >= 0: waits.append(float(r.get("waitingTime", 0)))
    except Exception: pass
    for f in (tri, persons):
        try: os.remove(f)
        except OSError: pass
    out = dict(headway_cv=np.mean(cvs) if cvs else float("nan"),
               travel_s=np.mean(tt) if tt else float("nan"),
               wait_s=num / den if den else float("nan"),
               wait_direct=float(np.mean(waits)) if waits else float("nan"))
    if trace:
        out["traj"] = {b: sorted([(barr[(b, i)], i) for i in range(ns) if (b, i) in barr])
                       for b in range(NBUS)}     # per-bus [(arrival_time, stop_index), ...]
        out["cum"] = [sum(DIST[:i]) for i in range(ns)]                   # cumulative distance per stop (m)
    return out
 
 
if __name__ == "__main__":
    # Parity check: NC/FH/EH via simulate() at ALL interior control stops, D+T (Stage A), should
    # reproduce the committed MC (NC~0.335, FH~0.153, EH~0.172 headway CV).
    import numpy as np
    print("controller | mean headway CV (D+T, all-interior control stops, seeds 0-2)")
    for name, fn in BASELINES.items():
        cvs = [simulate(fn, seed=s, T=True)["headway_cv"] for s in range(3)]
        print(f"   {name:2s}      |  {np.mean(cvs):.3f}   (per-seed {[round(c,3) for c in cvs]})")

## scripts/run_baseline.py

Single-seed NC/FH/EH under ideal demand; direct vs model wait cross-check.

"""Baseline evaluation harness (SO3): No-Control vs Even-Headway, with passenger demand.
 
A fleet runs the calibrated corridor at scheduled headway H0 with stochastic dwell (stand-in
for demand variability that induces bunching). Passengers are injected at each stop at the
empirical APC boarding rate and ride to the last stop; SUMO records each rider's wait.
No-Control: buses run freely. Even-Headway: a bus whose forward headway < H0 is held
(Rodriguez et al., 0.4*H cap). Departures are controlled with resume() so both boarding
(full stop list known) and holding work together.
 
Metrics: headway CV (bunching), mean corridor travel time, mean & SD passenger waiting time.
Self-contained: generates sumo/vtype.add.xml and sumo/persons.add.xml. Run from repo root;
needs sumo/corridor.net.xml + sumo/stops.add.xml + sim_inputs/stops.csv.
Note: mean wait scales as (H0/2)(1+CV^2), so EH's waiting benefit grows with bunching severity.
"""
import os, sys, numpy as np, pandas as pd, xml.etree.ElementTree as ET
if "SUMO_HOME" in os.environ:
    sys.path.insert(0, os.path.join(os.environ["SUMO_HOME"], "tools"))
import traci
from sumolib import checkBinary
 
STOPS = [l.strip() for l in open("corridor.txt") if l.strip()]   # full 26-stop dir-6 corridor
EDGES = [f"e{i}" for i in range(len(STOPS))]
H0, NBUS, CV_DWELL, CAP, BIG = 300.0, 12, 0.25, 0.4, 600.0
FIXED_DWELL, BOARD_S, MAX_SERV = 6.0, 4.0, 90.0   # dwell = door time + boarding_s * waiting pax (capped)
 
d = pd.read_csv("sim_inputs/stops.csv").set_index("bs_id").loc[[int(s) for s in STOPS]]
BASE   = {STOPS[i]: max(8.0, float(d["dwell_s"].values[i]))        for i in range(len(STOPS))}
DEMAND = {STOPS[i]: max(0.0, float(d["mean_boardings"].values[i])) for i in range(len(STOPS))}
 
os.makedirs("sumo", exist_ok=True)
open("sumo/vtype.add.xml", "w").write(
    '<additional><vType id="bus" vClass="bus" length="12" accel="1.2" decel="4.0" '
    'maxSpeed="30" personCapacity="60"/></additional>\n')
# Align each stop's demand window with when buses actually serve it: a bus leaving the
# terminal at t reaches stop i after CUM[i] (cumulative run + upstream dwell). Injecting
# uniformly over [CUM[i], CUM[i] + H0*(NBUS-1)] keeps waits at steady-state ~H0/2 instead of
# inflating far-stop waits with riders who arrive before the first / after the last bus.
RUN = [float(d["run_s"].values[i]) for i in range(len(STOPS))]
CUM = [0.0]
for i in range(1, len(STOPS)):
    CUM.append(CUM[-1] + RUN[i - 1] + BASE[STOPS[i - 1]])
p = ["<additional>"]
for i in range(len(STOPS) - 1):
    K = int(round(NBUS * DEMAND[STOPS[i]]))
    if K > 0:
        b, e = int(CUM[i]), int(CUM[i] + H0 * (NBUS - 1))
        p.append(f'  <personFlow id="f{i}" begin="{b}" end="{e}" number="{K}">'
                 f'<stop busStop="{STOPS[i]}" duration="1"/>'
                 f'<ride busStop="{STOPS[-1]}" lines="801"/></personFlow>')
p.append("</additional>")
open("sumo/persons.add.xml", "w").write("\n".join(p))
 
def fh_hold(hf):                       # Forward-Headway: hold up to target headway behind the leader
    return 0.0 if hf >= H0 else float(min(H0 - hf, CAP * H0))
def eh_hold(hf, hb):                    # Even-Headway (two-way): center the bus between leader and follower
    return float(max(0.0, min(0.5 * (hb - hf), CAP * H0)))
 
def run(controller, seed=1):
    rng = np.random.default_rng(seed); tri = f"sumo/tripinfo_{controller}.xml"
    traci.start([checkBinary("sumo"), "-n", "sumo/corridor.net.xml",
                 "-a", "sumo/vtype.add.xml,sumo/stops.add.xml,sumo/persons.add.xml",
                 "--tripinfo-output", tri, "--no-warnings", "true", "--no-step-log", "true",
                 "--step-length", "1", "-e", "24000"])
    traci.route.add("corr", EDGES)
    departs = {f"b{i}": i * H0 for i in range(NBUS)}
    added, toinit = set(), set()
    idx, prev, tarr, target, resumed, entry, corr_arr, dwell = {}, {}, {}, {}, set(), {}, {}, {}
    arr = {s: [] for s in STOPS}; barr = {}
    t = 0.0
    while t < H0 * NBUS + 20000 and (traci.simulation.getMinExpectedNumber() > 0 or len(added) < NBUS):
        for v, dep in departs.items():
            if v not in added and t >= dep:
                traci.vehicle.add(v, "corr", typeID="bus", line="801"); added.add(v); toinit.add(v)
        traci.simulationStep(); t = traci.simulation.getTime(); live = set(traci.vehicle.getIDList())
        for v in list(toinit):
            if v in live:
                idx[v], prev[v], entry[v] = 0, False, t
                for k in range(len(STOPS)):
                    try: traci.vehicle.setBusStop(v, STOPS[k], duration=BIG)   # long stops; resume() controls departure
                    except traci.TraCIException: pass
                toinit.discard(v)
        for v in list(added):
            if v not in live or v in toinit: continue
            st, i = traci.vehicle.isStopped(v), idx[v]
            if st and not prev[v] and i < len(STOPS):                          # arrived at STOPS[i]
                s = STOPS[i]; arr[s].append(t); tarr[v] = t; bi = int(v[1:]); barr[(bi, i)] = t
                if i == len(STOPS) - 1: corr_arr[v] = t
                try: nwait = traci.busstop.getPersonCount(s)                    # demand-driven dwell
                except traci.TraCIException: nwait = 0
                dserv = min(MAX_SERV, FIXED_DWELL + BOARD_S * nwait) * rng.lognormal(0, CV_DWELL)
                hold = 0.0
                if controller in ("FH", "EH") and 0 < i < len(STOPS) - 1 and bi > 0:
                    hf = t - barr[(bi-1, i)] if (bi-1, i) in barr else H0
                    if controller == "FH":
                        hold = fh_hold(hf)
                    else:
                        hb = (barr[(bi, i-1)] - barr[(bi+1, i-1)]) if ((bi+1, i-1) in barr and (bi, i-1) in barr) else H0
                        hold = eh_hold(hf, hb)
                target[v] = max(FIXED_DWELL, dserv) + hold
            if st and v not in resumed and (t - tarr.get(v, t)) >= target.get(v, 0):
                try: traci.vehicle.resume(v); resumed.add(v)
                except traci.TraCIException: pass
            if (not st) and prev[v] and i < len(STOPS):
                idx[v] = i + 1; resumed.discard(v)
            prev[v] = st
    traci.close()
    cvs = [np.std(np.diff(sorted(arr[s]))) / np.mean(np.diff(sorted(arr[s]))) for s in STOPS[1:] if len(arr[s]) >= 3]
    tt  = [corr_arr[v] - entry[v] for v in corr_arr if v in entry]
    waits = []
    for pi in ET.parse(tri).getroot().findall("personinfo"):
        for ride in pi.findall("ride"):
            if float(ride.get("arrival", "-1")) >= 0: waits.append(float(ride.get("waitingTime", 0)))
    # analytical wait from simulated headways (the (H0/2)(1+CV^2) random-arrival model), boardings-weighted
    num, den = 0.0, 0.0
    for s in STOPS[1:]:
        h = np.diff(sorted(arr[s]))
        if len(h) >= 2 and DEMAND[s] > 0:
            Hb = h.mean(); cv = h.std() / Hb
            num += DEMAND[s] * 0.5 * Hb * (1 + cv * cv); den += DEMAND[s]
    return dict(headway_cv=np.mean(cvs) if cvs else float("nan"),
                travel_s=np.mean(tt) if tt else float("nan"),
                wait_mean=np.mean(waits) if waits else float("nan"),
                wait_sd=np.std(waits) if waits else float("nan"), boarded=len(waits),
                wait_hw=num / den if den else float("nan"))
 
if __name__ == "__main__":
    print("controller | headway CV | travel (s) | wait direct (s) | wait model (s) | boarded")
    R = {}
    for c in ["NC", "FH", "EH"]:
        r = run(c); R[c] = r
        print(f"   {c:3s}    |   {r['headway_cv']:6.3f}   |   {r['travel_s']:6.1f}  |     {r['wait_mean']:6.1f}      |     {r['wait_hw']:6.1f}     |  {r['boarded']}")
    for c in ["FH", "EH"]:
        print(f"\n{c} vs NC: headway CV {(R[c]['headway_cv']-R['NC']['headway_cv'])/R['NC']['headway_cv']*100:+.0f}%, "
              f"travel {(R[c]['travel_s']-R['NC']['travel_s'])/R['NC']['travel_s']*100:+.0f}%, "
              f"wait(model) {(R[c]['wait_hw']-R['NC']['wait_hw'])/R['NC']['wait_hw']*100:+.0f}%")

## scripts/run_disturbances.py

D/S/T/W/B generators + single-seed scenario table.

"""Disturbance generators (SO1.2) — full D/S/T/W/B suite + scenario comparison (NC vs EH).
 
Generators (toggle per run):
  D  baseline stochastic demand — always on (persons injected at empirical APC boarding rates).
  S  demand surge — a burst of extra passengers at a mid-corridor stop over a time window.
  T  traffic-speed — per-segment speed factor U(0.8, 1.2)  (Wangsun clip).
  W  weather — per-segment heavy-tailed factor F_W~LogNormal, E=1, CV=eta, capped [0.5,3] (Patil et al.).
  B  breakdown — one bus immobilized mid-corridor for TBREAK s (Cao et al.).
 
Speed factors combine as segment_speed / (F_W * (1/F_T)); dwell/hold via resume() so boarding
and holding coexist. Metrics: headway CV, corridor travel time, mean passenger wait.
Run from repo root; needs sumo/corridor.net.xml, sumo/stops.add.xml, sim_inputs/{stops,stop_coordinates}.csv.
"""
import os, sys, math, numpy as np, pandas as pd, xml.etree.ElementTree as ET
if "SUMO_HOME" in os.environ:
    sys.path.insert(0, os.path.join(os.environ["SUMO_HOME"], "tools"))
import traci, sumolib
from sumolib import checkBinary
 
STOPS = [l.strip() for l in open("corridor.txt") if l.strip()]   # full 26-stop dir-6 corridor
EDGES = [f"e{i}" for i in range(len(STOPS))]
co = pd.read_csv("sim_inputs/stop_coordinates.csv").set_index("bs_id").loc[[int(s) for s in STOPS]]
d  = pd.read_csv("sim_inputs/stops.csv").set_index("bs_id").loc[[int(s) for s in STOPS]]
lat0, lon0 = co["mean_lat"].mean(), co["mean_lon"].mean()
Xs = (co["mean_lon"] - lon0) * math.cos(math.radians(lat0)) * 111320
Ys = (co["mean_lat"] - lat0) * 110540
DIST = [math.hypot(Xs.values[i+1]-Xs.values[i], Ys.values[i+1]-Ys.values[i]) for i in range(len(STOPS)-1)]
SEGV = [DIST[i] / d["run_s"].values[i] for i in range(len(DIST))]
BASE = {STOPS[i]: max(8.0, float(d["dwell_s"].values[i]))        for i in range(len(STOPS))}
DEM  = {STOPS[i]: max(0.0, float(d["mean_boardings"].values[i])) for i in range(len(STOPS))}
H0, NBUS, CVD, CAP, BIG, TBREAK, ETA = 300.0, 12, 0.25, 0.4, 600.0, 400.0, 0.8
FIXED_DWELL, BOARD_S, MAX_SERV = 6.0, 4.0, 90.0   # dwell = door time + boarding_s * waiting pax (capped)
# CUM[i] = time for a bus leaving the terminal at t=0 to reach stop i (run + upstream dwell);
# used to align each stop's demand window with when buses actually serve it (steady-state waits).
RUN = [float(d["run_s"].values[i]) for i in range(len(STOPS))]
CUM = [0.0]
for i in range(1, len(STOPS)):
    CUM.append(CUM[-1] + RUN[i - 1] + BASE[STOPS[i - 1]])
SURGE_I = len(STOPS) // 3                              # an upper-corridor stop, so the surge has room to propagate
 
def fh_hold(hf):                       # Forward-Headway: hold up to target headway behind the leader
    return 0.0 if hf >= H0 else float(min(H0 - hf, CAP * H0))
def eh_hold(hf, hb):                    # Even-Headway (two-way): center the bus between leader and follower
    return float(max(0.0, min(0.5 * (hb - hf), CAP * H0)))
def w_factor(rng): s2 = math.log(1 + ETA*ETA); return float(min(3.0, max(0.5, rng.lognormal(-0.5*s2, math.sqrt(s2)))))
def t_factor(rng): return float(rng.uniform(0.8, 1.2))
def make_persons(S):
    p = ["<additional>"]
    for i in range(len(STOPS)-1):
        K = int(round(NBUS * DEM[STOPS[i]]))
        if K > 0:
            b, e = int(CUM[i]), int(CUM[i] + H0*(NBUS-1))
            p.append(f'<personFlow id="f{i}" begin="{b}" end="{e}" number="{K}">'
                     f'<stop busStop="{STOPS[i]}" duration="1"/><ride busStop="{STOPS[-1]}" lines="801"/></personFlow>')
    if S:  # sustained demand surge at an upper-corridor stop over ~3 headways (overflow propagates downstream)
        sb = int(CUM[SURGE_I] + H0)
        p.append(f'<personFlow id="surge" begin="{sb}" end="{sb+900}" number="120">'
                 f'<stop busStop="{STOPS[SURGE_I]}" duration="1"/><ride busStop="{STOPS[-1]}" lines="801"/></personFlow>')
    open("sumo/persons_x.add.xml", "w").write("\n".join(p + ["</additional>"]))
 
def run(ctrl, D=True, S=False, T=False, W=False, B=False, seed=1):
    rng = np.random.default_rng(1000 + seed); make_persons(S)
    # Pre-draw every disturbance as a fixed field per (bus, stop) so NC and EH at the same seed
    # experience the IDENTICAL realization -> a truly paired comparison (controller-independent).
    ns = len(STOPS)
    DWN = rng.lognormal(0.0, CVD, size=(NBUS, ns))                       # dwell noise
    s2 = math.log(1 + ETA * ETA)
    WF = np.clip(rng.lognormal(-0.5 * s2, math.sqrt(s2), size=(NBUS, ns)), 0.5, 3.0)  # weather factor
    TF = rng.uniform(0.8, 1.2, size=(NBUS, ns))                          # traffic factor
    bk_idx = int(rng.integers(2, NBUS - 1)); bks = int(rng.integers(1, ns - 1)) if B else -1
    tri = "sumo/tri_x.xml"; port = sumolib.miscutils.getFreeSocketPort()
    traci.start([checkBinary("sumo"), "-n", "sumo/corridor.net.xml",
                 "-a", "sumo/vtype.add.xml,sumo/stops.add.xml,sumo/persons_x.add.xml",
                 "--tripinfo-output", tri, "--no-warnings", "true", "--no-step-log", "true",
                 "--seed", str(seed), "--step-length", "1", "-e", "36000"], port=port)
    traci.route.add("corr", EDGES)
    departs = {f"b{i}": i*H0 for i in range(NBUS)}
    added, toinit, idx, prev = set(), set(), {}, {}
    arr = {s: [] for s in STOPS}; tarr, target, resumed, entry, ca, dwell = {}, {}, set(), {}, {}, {}
    barr = {}                          # (bus_index, stop_index) -> arrival time, for leader/follower headways
    t = 0.0
    while t < H0*NBUS + 30000 and (traci.simulation.getMinExpectedNumber() > 0 or len(added) < NBUS):
        for v, dep in departs.items():
            if v not in added and t >= dep:
                traci.vehicle.add(v, "corr", typeID="bus", line="801"); added.add(v); toinit.add(v)
        traci.simulationStep(); t = traci.simulation.getTime(); live = set(traci.vehicle.getIDList())
        for v in list(toinit):
            if v in live:
                idx[v], prev[v], entry[v] = 0, False, t
                for k in range(len(STOPS)):
                    try: traci.vehicle.setBusStop(v, STOPS[k], duration=BIG)
                    except traci.TraCIException: pass
                toinit.discard(v)
        for v in list(added):
            if v not in live or v in toinit: continue
            st, i = traci.vehicle.isStopped(v), idx[v]
            if st and not prev[v] and i < len(STOPS):
                s = STOPS[i]; arr[s].append(t); tarr[v] = t
                if i == len(STOPS)-1: ca[v] = t
                bi = int(v[1:]); barr[(bi, i)] = t
                try: nwait = traci.busstop.getPersonCount(s)          # passengers waiting -> demand-driven dwell
                except traci.TraCIException: nwait = 0
                dserv = min(MAX_SERV, FIXED_DWELL + BOARD_S * nwait) * DWN[bi, i]
                hold = 0.0
                if ctrl in ("FH", "EH") and 0 < i < len(STOPS)-1 and bi > 0:
                    hf = t - barr[(bi-1, i)] if (bi-1, i) in barr else H0             # gap to leader (this stop)
                    if ctrl == "FH":
                        hold = fh_hold(hf)
                    else:                                                            # EH: gap to follower via previous stop
                        hb = (barr[(bi, i-1)] - barr[(bi+1, i-1)]) if ((bi+1, i-1) in barr and (bi, i-1) in barr) else H0
                        hold = eh_hold(hf, hb)
                tb = TBREAK if (B and bi == bk_idx and i == bks) else 0.0
                target[v] = max(FIXED_DWELL, dserv) + hold + tb
                try: traci.vehicle.setMaxSpeed(v, 30.0)
                except traci.TraCIException: pass
            if st and v not in resumed and (t - tarr.get(v, t)) >= target.get(v, 0):
                try: traci.vehicle.resume(v); resumed.add(v)
                except traci.TraCIException: pass
                if i < len(DIST):
                    f = 1.0
                    if W: f *= float(WF[bi, i])
                    if T: f /= float(TF[bi, i])
                    if f != 1.0:
                        try: traci.vehicle.setMaxSpeed(v, max(2.0, SEGV[i] / f))
                        except traci.TraCIException: pass
            if (not st) and prev[v] and i < len(STOPS): idx[v] = i + 1; resumed.discard(v)
            prev[v] = st
    traci.close()
    cvs = [np.std(np.diff(sorted(arr[s]))) / np.mean(np.diff(sorted(arr[s]))) for s in STOPS[1:] if len(arr[s]) >= 3]
    tt  = [ca[v] - entry[v] for v in ca if v in entry]
    # Passenger wait from the simulated bus-arrival headways via the random-arrival model
    # w = (E[H]/2)(1 + CV^2), boardings-weighted over stops (robust to demand-injection timing;
    # this is the same (H0/2)(1+CV^2) relationship stated in the methods).
    num, den = 0.0, 0.0
    for s in STOPS[1:]:
        h = np.diff(sorted(arr[s]))
        if len(h) >= 2 and DEM[s] > 0:
            Hb = h.mean(); cv = h.std() / Hb
            num += DEM[s] * 0.5 * Hb * (1 + cv * cv); den += DEM[s]
    wait = num / den if den else float("nan")
    return (np.mean(cvs) if cvs else float("nan"), np.mean(tt) if tt else float("nan"), wait)
 
if __name__ == "__main__":
    scen = [("D baseline", {}), ("S surge", dict(S=True)), ("T traffic", dict(T=True)),
            ("W weather", dict(W=True)), ("B breakdown", dict(B=True)),
            ("all D/S/T/W/B", dict(S=True, T=True, W=True, B=True))]
    print("scenario        | ctrl | headwayCV | travel(s) | wait(s)")
    for name, kw in scen:
        for c in ["NC", "EH"]:
            cv, tt, w = run(c, **kw)
            print(f"{name:15s} | {c:2s}   |  {cv:6.3f}  |   {tt:6.0f}  | {w:6.1f}")

## scripts/mc.py

Parallel N-seed Monte-Carlo over the activation matrix; bootstrap CIs.

"""Monte-Carlo evaluation (SO3): NC / FH / EH on the corridor, manuscript activation matrix.
 
Drives the unified core `envs/corridor_sim.simulate(decide, control_stops, ...)` so all controllers
act at the DESIGNATED CONTROL STOPS (four §3.2.2 criteria) — the same stops the MARL agent will use,
keeping the comparison fair. N paired replications per cell (seed s = identical disturbance across
controllers). Records headway CV, travel time, and wait as both the headway-model estimate (`wait_s`,
primary/robust) and SUMO's per-passenger recording (`wait_direct`, cross-check). Bootstrap 95% CIs +
paired % change vs No-Control.
 
Runs in PARALLEL across processes (SUMO is CPU-bound and single-threaded per instance, so this is the
speed lever). Run from the repo root:
    python scripts/mc.py            # N=30, jobs=6
    python scripts/mc.py 30 8       # N=30, 8 parallel workers
    python scripts/mc.py 20 1       # serial
Writes results/mc_results.csv and results/mc_summary.md.
"""
import os, sys, time, csv, numpy as np
from concurrent.futures import ProcessPoolExecutor, as_completed
sys.path.insert(0, os.path.join(os.path.dirname(os.path.abspath(__file__)), "..", "envs"))
from corridor_sim import simulate, BASELINES, CONTROL_STOPS, STOPS
 
N    = int(sys.argv[1]) if len(sys.argv) > 1 else 30
JOBS = int(sys.argv[2]) if len(sys.argv) > 2 else 6
SCEN = [("Stage A (D+T)",        dict(T=True)),
        ("Ablation S (D+T+S)",   dict(T=True, S=True)),
        ("Ablation W (D+T+W)",   dict(T=True, W=True)),
        ("Ablation B (D+T+B)",   dict(T=True, B=True)),
        ("Stage B (D+T+S+W+B)",  dict(T=True, S=True, W=True, B=True))]
CTRLS = list(BASELINES)              # NC, FH, EH
os.makedirs("results", exist_ok=True)
 
 
def _run_one(task):
    """Worker: one (scenario, controller, seed) replication. Picklable — looks up the decide fn locally."""
    name, kw, c, seed = task
    try:
        r = simulate(BASELINES[c], seed=seed, control_stops=CONTROL_STOPS, **kw)
        return (name, c, seed, r["headway_cv"], r["travel_s"], r["wait_s"], r["wait_direct"])
    except Exception:
        return (name, c, seed, float("nan"), float("nan"), float("nan"), float("nan"))
 
 
def boot_ci(x, f=np.mean, n=5000, rng=np.random.default_rng(0)):
    x = np.asarray([v for v in x if np.isfinite(v)])
    if len(x) < 2: return (float("nan"), float("nan"), float("nan"))
    bs = [f(rng.choice(x, len(x), replace=True)) for _ in range(n)]
    return float(f(x)), float(np.percentile(bs, 2.5)), float(np.percentile(bs, 97.5))
 
 
def paired_pct(nc, eh, n=5000, rng=np.random.default_rng(1)):
    nc, eh = np.asarray(nc), np.asarray(eh); m = np.isfinite(nc) & np.isfinite(eh)
    nc, eh = nc[m], eh[m]
    if len(nc) < 2: return (float("nan"), float("nan"), float("nan"))
    pt = (eh.mean() - nc.mean()) / nc.mean() * 100
    bs = [((eh[i].mean() - nc[i].mean()) / nc[i].mean() * 100)
          for i in (rng.integers(0, len(nc), len(nc)) for _ in range(n))]
    return float(pt), float(np.percentile(bs, 2.5)), float(np.percentile(bs, 97.5))
 
 
def main():
    t0 = time.time()
    print(f"control stops: {[STOPS[i] for i in CONTROL_STOPS]}  (N={N}, jobs={JOBS})", flush=True)
    tasks = [(name, kw, c, s) for name, kw in SCEN for c in CTRLS for s in range(N)]
    rows = []
    with open("results/mc_results.csv", "w", newline="") as fh:
        w = csv.writer(fh)
        w.writerow(["scenario", "controller", "seed", "headway_cv", "travel_s", "wait_s", "wait_direct"])
        if JOBS > 1:
            # persistent workers (no max_tasks_per_child: with a balanced pool all workers hit the
            # recycle limit simultaneously and deadlock on Windows). Memory stays flat because each
            # run opens+closes its own SUMO subprocess, so nothing accumulates in the worker.
            with ProcessPoolExecutor(max_workers=JOBS) as ex:
                futs = [ex.submit(_run_one, t) for t in tasks]
                for k, fut in enumerate(as_completed(futs), 1):
                    row = fut.result(); rows.append(row); w.writerow(row); fh.flush()
                    if k % 15 == 0 or k == len(tasks):
                        print(f"  {k}/{len(tasks)} runs done  ({time.time()-t0:.0f}s)", flush=True)
        else:
            for k, t in enumerate(tasks, 1):
                row = _run_one(t); rows.append(row); w.writerow(row); fh.flush()
                if k % 15 == 0 or k == len(tasks):
                    print(f"  {k}/{len(tasks)} runs done  ({time.time()-t0:.0f}s)", flush=True)
 
    D = {}
    for name, c, s, cv, tt, wt, wd in rows:
        D.setdefault((name, c), {"cv": [], "tt": [], "wt": [], "wd": []})
        D[(name, c)]["cv"].append(cv); D[(name, c)]["tt"].append(tt)
        D[(name, c)]["wt"].append(wt); D[(name, c)]["wd"].append(wd)
    L = [f"Control stops: {[STOPS[i] for i in CONTROL_STOPS]} (§3.2.2 criteria). "
         f"Wait = headway model; wait_dir = SUMO per-passenger (cross-check).", "",
         "| Scenario | Ctrl | Headway CV [95% CI] | Travel (s) [95% CI] | Wait (s) [95% CI] | wait_dir | n |",
         "|---|---|---|---|---|--:|--:|"]
    for name, _ in SCEN:
        for c in CTRLS:
            d = D[(name, c)]
            cv = boot_ci(d["cv"]); tt = boot_ci(d["tt"]); wt = boot_ci(d["wt"]); wd = boot_ci(d["wd"])
            n = sum(np.isfinite(v) for v in d["cv"])
            L.append(f"| {name} | {c} | {cv[0]:.3f} [{cv[1]:.3f}, {cv[2]:.3f}] | "
                     f"{tt[0]:.0f} [{tt[1]:.0f}, {tt[2]:.0f}] | {wt[0]:.0f} [{wt[1]:.0f}, {wt[2]:.0f}] | "
                     f"{wd[0]:.0f} | {n} |")
    L += ["", "**Paired % change vs No-Control (negative = controller better; CV with 95% CI):**", "",
          "| Scenario | FH Δ CV % [95% CI] | FH Δ wait % | EH Δ CV % [95% CI] | EH Δ wait % |",
          "|---|---|---|---|---|"]
    for name, _ in SCEN:
        fcv = paired_pct(D[(name, "NC")]["cv"], D[(name, "FH")]["cv"])
        fwt = paired_pct(D[(name, "NC")]["wt"], D[(name, "FH")]["wt"])
        ecv = paired_pct(D[(name, "NC")]["cv"], D[(name, "EH")]["cv"])
        ewt = paired_pct(D[(name, "NC")]["wt"], D[(name, "EH")]["wt"])
        L.append(f"| {name} | {fcv[0]:+.0f}% [{fcv[1]:+.0f}, {fcv[2]:+.0f}] | {fwt[0]:+.0f}% | "
                 f"{ecv[0]:+.0f}% [{ecv[1]:+.0f}, {ecv[2]:+.0f}] | {ewt[0]:+.0f}% |")
    open("results/mc_summary.md", "w", encoding="utf-8").write("\n".join(L) + "\n")
    print("\n".join(L).encode("ascii", "replace").decode())
    print(f"\nMC done: {len(SCEN)}x{len(CTRLS)}x{N} runs in {time.time()-t0:.0f}s -> results/mc_summary.md", flush=True)
 
 
if __name__ == "__main__":
    main()

## scripts/figures.py

Calibration and Monte-Carlo figures.

"""Generate the paper figures from saved results (full 26-stop corridor).
 
Reads results/calibration.csv (from calibrate_corridor.py) and results/mc_results.csv (from mc.py)
and writes PNGs to results/figures/:
  calibration_validation.png  simulated vs observed segment running times (GEH<5)
  mc_headway_cv.png           headway CV by scenario, NC vs EH, 95% bootstrap CI
  mc_wait.png                 passenger wait by scenario, NC vs EH, 95% bootstrap CI
Run from the repo root after calibrate_corridor.py and mc.py.
"""
import os, numpy as np, pandas as pd
import matplotlib
matplotlib.use("Agg")
import matplotlib.pyplot as plt
 
os.makedirs("results/figures", exist_ok=True)
SCEN  = ["Stage A (D+T)", "Ablation S (D+T+S)", "Ablation W (D+T+W)", "Ablation B (D+T+B)", "Stage B (D+T+S+W+B)"]
SHORT = ["Stage A\n(D+T)", "Abl. S\n(+surge)", "Abl. W\n(+weather)", "Abl. B\n(+breakdown)", "Stage B\n(all)"]
CTRLS = [("NC", "#9e9e9e"), ("FH", "#e0913a"), ("EH", "#1f63d6")]     # No-Control, Forward-, Even-Headway
BLUE, GREY = "#1f63d6", "#9e9e9e"
 
 
def boot(x, f=np.mean, n=5000, rng=np.random.default_rng(0)):
    x = np.asarray([v for v in x if np.isfinite(v)])
    if len(x) < 2: return (np.nan, np.nan, np.nan)
    bs = [f(rng.choice(x, len(x), True)) for _ in range(n)]
    return float(f(x)), float(np.percentile(bs, 2.5)), float(np.percentile(bs, 97.5))
 
 
def calibration_fig():
    if not os.path.exists("results/calibration.csv"): return
    c = pd.read_csv("results/calibration.csv")
    fig, ax = plt.subplots(figsize=(6, 6))
    lim = max(c.observed_s.max(), c.simulated_s.max()) * 1.08
    ax.plot([0, lim], [0, lim], "--", color=GREY, lw=1, label="perfect match")
    ax.scatter(c.observed_s, c.simulated_s, s=45, color=BLUE, zorder=3, edgecolor="white")
    rmspe = np.sqrt(np.mean(((c.simulated_s - c.observed_s) / c.observed_s) ** 2)) * 100
    ax.set_xlabel("Observed segment running time (s, APC)")
    ax.set_ylabel("Simulated segment running time (s, SUMO)")
    ax.set_title(f"Corridor calibration: {len(c)} segments, all GEH<5, RMSPE {rmspe:.1f}%")
    ax.set_xlim(0, lim); ax.set_ylim(0, lim); ax.legend(loc="upper left"); ax.grid(alpha=.3)
    fig.tight_layout(); fig.savefig("results/figures/calibration_validation.png", dpi=150); plt.close(fig)
    print("wrote calibration_validation.png")
 
 
def scenario_fig(df, col, ylabel, title, fname):
    x = np.arange(len(SCEN)); n = len(CTRLS); wpx = 0.8 / n
    fig, ax = plt.subplots(figsize=(10, 5.2))
    for k, (c, color) in enumerate(CTRLS):
        off = (k - (n - 1) / 2) * wpx
        m, lo, hi = [], [], []
        for sc in SCEN:
            v = df[(df.scenario == sc) & (df.controller == c)][col].values
            pm, plo, phi = boot(v); m.append(pm); lo.append(pm - plo); hi.append(phi - pm)
        ax.bar(x + off, m, wpx, yerr=[lo, hi], capsize=2.5, color=color, label=c,
               error_kw=dict(lw=1, ecolor="#333"))
    ax.set_xticks(x); ax.set_xticklabels(SHORT); ax.set_ylabel(ylabel); ax.set_title(title)
    ax.legend(title="Controller"); ax.grid(axis="y", alpha=.3); ax.set_axisbelow(True)
    fig.tight_layout(); fig.savefig(f"results/figures/{fname}", dpi=150); plt.close(fig)
    print(f"wrote {fname}")
 
 
def main():
    calibration_fig()
    if os.path.exists("results/mc_results.csv"):
        df = pd.read_csv("results/mc_results.csv")
        scenario_fig(df, "headway_cv", "Headway CV (bunching)",
                     "Headway irregularity by scenario (NC / FH / EH), 95% CI", "mc_headway_cv.png")
        scenario_fig(df, "wait_s", "Mean passenger wait (s)",
                     "Passenger wait by scenario (NC / FH / EH), 95% CI", "mc_wait.png")
 
 
if __name__ == "__main__":
    main()

## scripts/marey.py

Time-space (Marey) trajectory diagram.

"""Marey (time-space) diagram — bus trajectories over the corridor. Bunching shows as converging lines.
 
A 2x2 panel: No-Control vs Even-Headway, each under Stage A (D+T) and Weather (D+T+W), at the designated
control stops. Uses one representative seed. -> results/figures/marey_diagram.png. Run from repo root.
"""
import os, sys, numpy as np
import matplotlib; matplotlib.use("Agg")
import matplotlib.pyplot as plt
sys.path.insert(0, "envs")
import corridor_sim as C
 
SEED = 5
CS = C.CONTROL_STOPS
PANELS = [("Stage A  (D+T)", dict(T=True)), ("Weather  (D+T+W)", dict(T=True, W=True))]
CTRLS = [("No-Control", "NC"), ("Even-Headway", "EH")]
 
fig, axes = plt.subplots(len(PANELS), len(CTRLS), figsize=(11, 8), sharex=True, sharey=True)
for ri, (sname, kw) in enumerate(PANELS):
    for ci, (clabel, cname) in enumerate(CTRLS):
        ax = axes[ri][ci]
        out = C.simulate(C.BASELINES[cname], seed=SEED, control_stops=CS, trace=True, **kw)
        cum = np.array(out["cum"]) / 1000.0                       # km
        for i in CS:                                              # mark control stops
            ax.axhline(cum[i], color="#cbd5e1", lw=0.8, zorder=0)
        for b, traj in out["traj"].items():
            if len(traj) < 2: continue
            ts = [t / 60.0 for t, i in traj]; ys = [cum[i] for t, i in traj]
            ax.plot(ts, ys, lw=1.2, alpha=0.85)
        ax.set_title(f"{clabel}  —  {sname}", fontsize=11)
        if ci == 0: ax.set_ylabel("distance along corridor (km)")
        if ri == len(PANELS) - 1: ax.set_xlabel("time (min)")
        ax.grid(alpha=0.2)
fig.suptitle("Bus trajectories at the designated control stops (grey lines) — bunching = trajectories converging",
             fontsize=12.5)
fig.tight_layout(rect=[0, 0, 1, 0.96])
os.makedirs("results/figures", exist_ok=True)
fig.savefig("results/figures/marey_diagram.png", dpi=150)
print("wrote results/figures/marey_diagram.png")

## scripts/degradation.py

Headway CV vs weather intensity sweep.

"""Degradation curve — headway CV vs weather intensity, per controller (visualizes EO 3.2).
 
Sweeps the weather CV parameter eta and plots mean headway CV for NC / FH / EH at the designated control
stops, so the loss of control as conditions worsen is a curve. Parallel across processes.
-> results/figures/degradation_curve.png. Run from repo root:  python scripts/degradation.py [N] [JOBS]
"""
import os, sys, numpy as np
from concurrent.futures import ProcessPoolExecutor
sys.path.insert(0, os.path.join(os.path.dirname(os.path.abspath(__file__)), "..", "envs"))
import corridor_sim as C
# NOTE: matplotlib is imported inside main() ONLY — importing it at module level makes every spawned
# worker re-import it, and concurrent matplotlib/font-cache init crashes workers on Windows.
 
ETAS = [0.0, 0.2, 0.4, 0.6, 0.8, 1.0, 1.2]
N    = int(sys.argv[1]) if len(sys.argv) > 1 else 12
JOBS = int(sys.argv[2]) if len(sys.argv) > 2 else 4
CS = C.CONTROL_STOPS
COLORS = {"NC": "#9e9e9e", "FH": "#e0913a", "EH": "#1f63d6"}
 
 
def _run(task):
    c, e, s = task
    cv = C.simulate(C.BASELINES[c], seed=s, control_stops=list(CS), T=True, W=(e > 0), eta=e)["headway_cv"]
    return (c, e, cv)
 
 
def main():
    import time
    tasks = [(c, e, s) for c in ["NC", "FH", "EH"] for e in ETAS for s in range(N)]
    res = {}; t0 = time.time()
    if JOBS > 1:
        with ProcessPoolExecutor(max_workers=JOBS) as ex:          # workers never import matplotlib
            for k, (c, e, cv) in enumerate(ex.map(_run, tasks), 1):
                res.setdefault((c, e), []).append(cv)
                if k % 30 == 0: print(f"  {k}/{len(tasks)} ({time.time()-t0:.0f}s)", flush=True)
    else:                                                          # reliable serial fallback
        for k, t in enumerate(tasks, 1):
            c, e, cv = _run(t); res.setdefault((c, e), []).append(cv)
            if k % 15 == 0: print(f"  {k}/{len(tasks)} ({time.time()-t0:.0f}s)", flush=True)
    import matplotlib; matplotlib.use("Agg")                       # parent only
    import matplotlib.pyplot as plt
    fig, ax = plt.subplots(figsize=(8.5, 5.2))
    for c in ["NC", "FH", "EH"]:
        m = [np.nanmean(res[(c, e)]) for e in ETAS]
        sd = [np.nanstd(res[(c, e)]) / np.sqrt(N) for e in ETAS]
        ax.errorbar(ETAS, m, yerr=sd, fmt="-o", color=COLORS[c], label=c, capsize=3, lw=1.8)
    ax.set_xlabel("weather intensity  η  (coefficient of variation of the weather speed factor)")
    ax.set_ylabel("headway CV (bunching)")
    ax.set_title(f"Degradation of headway control with weather severity (D+T+W, N={N} per point)")
    ax.legend(title="Controller"); ax.grid(alpha=0.3); ax.set_axisbelow(True)
    fig.tight_layout(); os.makedirs("results/figures", exist_ok=True)
    fig.savefig("results/figures/degradation_curve.png", dpi=150)
    print("wrote results/figures/degradation_curve.png")
 
 
if __name__ == "__main__":
    main()

## envs/reward.py

Reward-term library (candidate forms) + compose() + action decode.

"""Reward library for the MARL agent (SO2 / EO2.1).
 
The manuscript fixes the STRUCTURE — a weighted sum of three non-positive penalty terms
(headway irregularity, passenger waiting, degenerate-skip) — and leaves the exact expressions and
weights as the implementation deliverable. So each term is a CANDIDATE function here; a Config picks
which form and what weights. Nothing is committed: you sweep `irr`/`wait`/`skip` and `w=(w1,w2,w3)`.
 
A reward is assigned to a bus's action a_prev, realized at its NEXT control-stop decision, using the
state that action produced (semi-MDP: transition (s_prev, a_prev, r, s_cur)). `compose(...)` returns the
scalar reward; `decode_action` maps the discrete action id to (hold_seconds, skip).
 
    r = −( w1·irr(...) + w2·wait(...) + w3·skip(...) )      [each term ≥ 0]
"""
Q_REF = 20.0                           # reference queue for normalization (tunable scale constant)
 
 
def decode_action(a, H0=300.0, dt=300.0):
    """action id 0..9 -> (hold_seconds, skip). alpha in {0,.1,.2,.3,.4}, scaled by dt (=ΔT); skip binary."""
    alpha = (a % 5) * 0.1
    skip = a // 5
    return alpha * dt, int(skip)
 
 
# --- candidate terms: term(prev, cur, ctx) -> penalty >= 0.  ctx has H0, hold, skip ---------------
def irr_dev(prev, cur, ctx):           # 1a: squared deviation of forward headway from schedule H0
    return ((cur["hf"] - ctx["H0"]) / ctx["H0"]) ** 2
def irr_even(prev, cur, ctx):          # 1b: squared forward/backward headway asymmetry (even-spacing)
    return ((cur["hf"] - cur["hb"]) / ctx["H0"]) ** 2
def irr_both(prev, cur, ctx):          # 1c: both, averaged
    return 0.5 * (((cur["hf"] - ctx["H0"]) / ctx["H0"]) ** 2 + ((cur["hb"] - ctx["H0"]) / ctx["H0"]) ** 2)
 
def wait_queue(prev, cur, ctx):        # 2b: at-stop wait ~ waiting riders x headway they endured
    return (cur["queue"] * cur["hf"]) / (Q_REF * ctx["H0"])
def wait_hold(prev, cur, ctx):         # 2a: in-vehicle delay from holding, weighted by onboard load
    return (ctx["hold"] / ctx["H0"]) * (prev["load"] / prev["cap"])
 
def skip_stranded(prev, cur, ctx):     # 3a: stranded riders (demand-aware)
    return ctx["skip"] * (prev["queue"] / Q_REF)
def skip_flat(prev, cur, ctx):         # 3b: flat discouragement per skip
    return float(ctx["skip"])
 
IRR  = {"dev": irr_dev, "even": irr_even, "both": irr_both}
WAIT = {"queue": wait_queue, "hold": wait_hold}
SKIP = {"stranded": skip_stranded, "flat": skip_flat}
 
 
def compose(prev, cur, a_prev, cfg):
    """Scalar reward for action a_prev, realized at the next decision (state cur, prior state prev).
    cfg: object/dict with .irr .wait .skip (keys above), .w=(w1,w2,w3), .H0, .dt (=ΔT)."""
    g = (lambda k: getattr(cfg, k) if not isinstance(cfg, dict) else cfg[k])
    hold, skip = decode_action(a_prev, g("H0"), g("dt"))
    ctx = {"H0": g("H0"), "hold": hold, "skip": skip}
    w1, w2, w3 = g("w")
    ir = IRR[g("irr")](prev, cur, ctx)
    wt = WAIT[g("wait")](prev, cur, ctx)
    sk = SKIP[g("skip")](prev, cur, ctx)
    return -(w1 * ir + w2 * wt + w3 * sk)
 
 
if __name__ == "__main__":
    cfg = dict(irr="dev", wait="queue", skip="stranded", w=(1.0, 0.5, 1.0), H0=300.0, dt=300.0)
    prev = dict(hf=300, hb=300, load=20, queue=5, cap=60)
    on_time = dict(hf=300, hb=300, queue=3, cap=60)     # perfect headway -> ~0 penalty
    bunched = dict(hf=120, hb=480, queue=12, cap=60)    # early + long back gap -> large penalty
    print("action decode 0,4,5,9:", [decode_action(a) for a in (0, 4, 5, 9)])
    print(f"reward on-time (a=0 hold): {compose(prev, on_time, 0, cfg):.3f}")
    print(f"reward bunched (a=0 hold): {compose(prev, bunched, 0, cfg):.3f}")
    print(f"reward bunched (a=9 max hold+skip): {compose(prev, bunched, 9, cfg):.3f}")

## envs/obs.py

Observation featuriser: obs dict → normalised 7-vector.

"""Observation featurizer — corridor_sim's obs dict -> the manuscript 7-vector (Table 3.6), normalized.
 
Features (all ~[0,1] so the shared network sees a consistent scale):
    0 stop index      idx/(n-1)          spatial location along the corridor
    1 forward headway hf/H0              gap to leader (1.0 = on schedule)
    2 backward headway hb/H0             estimated gap to follower
    3 onboard load    load/cap           occupancy
    4 waiting queue   queue/Q_REF        passengers waiting at the stop
    5 weather flag    (w-0.5)/2.5        realized speed-factor intensity (neutral 1.0 -> 0.2)
    6 breakdown flag  b                  downstream incident present (0/1)
 
`featurize(obs)` returns a float32 array of length OBS_DIM. Variants (e.g. + neighboring-trip info)
are added here behind a config flag; the default is the manuscript vector.
"""
import numpy as np
from reward import Q_REF
 
OBS_DIM = 7
 
 
def featurize(o):
    H0, cap, n = o["H0"], o["cap"], o["n"]
    return np.array([
        o["idx"] / max(1, n - 1),
        o["hf"] / H0,
        o["hb"] / H0,
        o["load"] / cap,
        o["queue"] / Q_REF,
        (o["w"] - 0.5) / 2.5,
        o["b"],
    ], dtype=np.float32)
 
 
if __name__ == "__main__":
    o = dict(hf=300, hb=300, load=20, queue=5, idx=5, n=26, H0=300.0, cap=60, bus=3, w=1.0, b=0.0)
    v = featurize(o)
    print("obs vector (len %d):" % len(v), np.round(v, 3))
    assert len(v) == OBS_DIM
    print("ok")

## envs/marl_env.py

Config (all knobs) + MarlController (semi-MDP transition assembly, parameter sharing).

"""MARL controller + Config — plugs the shared DDQN into corridor_sim as a decide() function.
 
corridor_sim calls `decide(obs) -> (hold_seconds, skip)` at each control-stop arrival. `MarlController`
is a stateful decide: for each bus it remembers its last (obs, action), and on the bus's NEXT decision
it computes the reward for that action (from the resulting state), pushes the transition
(s_prev, a_prev, r, s_cur) into the shared replay buffer, learns, then picks the new action. This is
the semi-MDP assembly; parameter sharing = one agent instance serves every bus.
 
`Config` is the single place every experiment knob lives — reward form/weights, ΔT, skip on/off,
DDQN hyperparameters, control stops, training budget. Copy it, change a field, run; commit nothing.
"""
from dataclasses import dataclass
import numpy as np
from obs import featurize, OBS_DIM
from reward import compose, decode_action
 
N_ACTIONS = 10
 
 
@dataclass
class Config:
    # reward (candidate forms from reward.py; weights are the EO2.1 sweep)
    irr: str = "dev"; wait: str = "queue"; skip: str = "stranded"
    w: tuple = (1.0, 0.5, 1.0)
    # action
    H0: float = 300.0; dt: float = 300.0; skip_enabled: bool = False
    # DDQN hyperparameters
    lr: float = 1e-3; gamma: float = 0.99
    eps_start: float = 1.0; eps_end: float = 0.05; eps_decay: int = 30_000
    buffer: int = 100_000; batch: int = 64; target_every: int = 500; warmup: int = 1_000
    net: tuple = (128, 128)
    # env / training
    control_stops: tuple = (0, 1, 5, 17, 20)
    episodes: int = 2_000
    seed: int = 0
    name: str = "gate"
 
 
class MarlController:
    """Use as the `decide` passed to corridor_sim.simulate. training=True explores + learns."""
    def __init__(self, agent, cfg, training=True):
        self.agent, self.cfg, self.training = agent, cfg, training
        self.prev = {}                 # bus -> (obs_vec, action, obs_dict)
        self.ret = 0.0; self.n = 0     # episode return and transition count (diagnostics)
 
    def __call__(self, obs):
        bi = obs["bus"]
        ov = featurize(obs)
        if bi in self.prev:
            pov, pa, pobs = self.prev[bi]
            r = compose(pobs, obs, pa, self.cfg)
            self.ret += r; self.n += 1
            if self.training:
                self.agent.push(pov, pa, r, ov, False)
                self.agent.learn()
        a = self.agent.act(ov, greedy=not self.training)
        self.prev[bi] = (ov, a, obs)
        hold, skip = decode_action(a, self.cfg.H0, self.cfg.dt)
        return hold, (skip if self.cfg.skip_enabled else 0)
 
    def finalize(self):
        """End of episode: drop each bus's last (obs, action) — it has no next state to bootstrap from."""
        self.prev.clear()

## agents/ddqn.py

Parameter-shared Double-DQN: shared net, replay, double-Q update, epsilon-greedy.

"""Parameter-shared Double DQN learner (SO2 core) — the shared network + replay + update rule.
 
One shared Q-network serves every bus (parameter sharing); each bus calls `act(obs)` with its own
local observation and stores its own `(o,a,r,o2,done)` transition in one shared replay buffer
(CTDE: centralized training on pooled per-agent transitions, decentralized execution). The update is
Double-DQN: the online net selects the next action, the target net evaluates it.
 
    y = r + γ (1-done) · Q(o2, argmaxₐ Q(o2,a; θ); θ⁻)
 
This module is the algorithm only — obs featurization (the 7-vector of Table 3.6), the reward, the
action decode (α×ΔT, skip), and the SUMO training loop live in the env/train layer. Hyperparameters
here are implementation defaults (EO2.1), tunable. Depends on torch (CPU is fine).
 
Self-test: `python agents/ddqn.py` trains on a synthetic contextual bandit and checks the shared net
learns to pick the rewarding action (accuracy climbs well above chance).
"""
from __future__ import annotations
import random, numpy as np, torch, torch.nn as nn
 
OBS_DIM, N_ACTIONS = 7, 10                     # manuscript: Table 3.6 obs; |A| = 5 holding × 2 skip
 
 
class QNet(nn.Module):
    def __init__(self, obs_dim=OBS_DIM, n_actions=N_ACTIONS, hidden=(128, 128)):
        super().__init__()
        layers, d = [], obs_dim
        for h in hidden:
            layers += [nn.Linear(d, h), nn.ReLU()]; d = h
        layers += [nn.Linear(d, n_actions)]
        self.net = nn.Sequential(*layers)
 
    def forward(self, x):
        return self.net(x)
 
 
class ReplayBuffer:
    def __init__(self, capacity, obs_dim=OBS_DIM):
        self.cap = capacity; self.i = 0; self.full = False
        self.o  = np.zeros((capacity, obs_dim), np.float32)
        self.a  = np.zeros(capacity, np.int64)
        self.r  = np.zeros(capacity, np.float32)
        self.o2 = np.zeros((capacity, obs_dim), np.float32)
        self.d  = np.zeros(capacity, np.float32)
 
    def push(self, o, a, r, o2, done):
        i = self.i
        self.o[i], self.a[i], self.r[i], self.o2[i], self.d[i] = o, a, r, o2, float(done)
        self.i = (i + 1) % self.cap
        self.full = self.full or self.i == 0
 
    def __len__(self):
        return self.cap if self.full else self.i
 
    def sample(self, batch):
        idx = np.random.randint(0, len(self), size=batch)
        t = lambda x: torch.as_tensor(x[idx])
        return t(self.o), t(self.a), t(self.r), t(self.o2), t(self.d)
 
 
class DDQNAgent:
    """Shared learner. All buses share this one instance: act(obs) per bus, push(...) per bus."""
    def __init__(self, obs_dim=OBS_DIM, n_actions=N_ACTIONS, lr=1e-3, gamma=0.99,
                 buffer=100_000, batch=64, target_every=500, warmup=1_000,
                 eps_start=1.0, eps_end=0.05, eps_decay=30_000, hidden=(128, 128),
                 device="cpu", seed=0):
        torch.manual_seed(seed); np.random.seed(seed); random.seed(seed)
        self.n_actions, self.gamma, self.batch = n_actions, gamma, batch
        self.target_every, self.warmup = target_every, warmup
        self.eps_start, self.eps_end, self.eps_decay = eps_start, eps_end, eps_decay
        self.device = torch.device(device)
        self.q  = QNet(obs_dim, n_actions, hidden).to(self.device)
        self.qt = QNet(obs_dim, n_actions, hidden).to(self.device)
        self.qt.load_state_dict(self.q.state_dict())
        self.opt = torch.optim.Adam(self.q.parameters(), lr=lr)
        self.buf = ReplayBuffer(buffer, obs_dim)
        self.steps = 0
 
    def epsilon(self):
        f = min(1.0, self.steps / self.eps_decay)
        return self.eps_start + f * (self.eps_end - self.eps_start)
 
    @torch.no_grad()
    def act(self, obs, greedy=False):
        if (not greedy) and random.random() < self.epsilon():
            return random.randrange(self.n_actions)
        o = torch.as_tensor(np.asarray(obs, np.float32), device=self.device).unsqueeze(0)
        return int(self.q(o).argmax(1).item())
 
    def push(self, o, a, r, o2, done):
        self.buf.push(np.asarray(o, np.float32), a, r, np.asarray(o2, np.float32), done)
 
    def learn(self):
        self.steps += 1
        if len(self.buf) < max(self.batch, self.warmup):
            return None
        o, a, r, o2, d = (x.to(self.device) for x in self.buf.sample(self.batch))
        q = self.q(o).gather(1, a.unsqueeze(1)).squeeze(1)
        with torch.no_grad():
            a2 = self.q(o2).argmax(1)                                   # online selects
            q2 = self.qt(o2).gather(1, a2.unsqueeze(1)).squeeze(1)      # target evaluates (double-Q)
            y = r + self.gamma * (1.0 - d) * q2
        loss = nn.functional.smooth_l1_loss(q, y)
        self.opt.zero_grad(); loss.backward()
        nn.utils.clip_grad_norm_(self.q.parameters(), 10.0)
        self.opt.step()
        if self.steps % self.target_every == 0:
            self.qt.load_state_dict(self.q.state_dict())
        return float(loss.item())
 
    def save(self, path): torch.save(self.q.state_dict(), path)
    def load(self, path): self.q.load_state_dict(torch.load(path, map_location=self.device)); self.qt.load_state_dict(self.q.state_dict())
 
 
if __name__ == "__main__":
    # Synthetic contextual bandit: reward 1 for the "correct" action (argmax of a fixed linear map of
    # obs), else 0. A working DDQN core should learn to pick it — accuracy climbs well above 1/10.
    rng = np.random.default_rng(0)
    W = rng.normal(size=(OBS_DIM, N_ACTIONS)).astype(np.float32)
    def best(o): return int(np.argmax(o @ W))
    ag = DDQNAgent(warmup=500, eps_decay=4000)
    correct, total = 0, 0
    for step in range(12_000):
        o = rng.normal(size=OBS_DIM).astype(np.float32)
        a = ag.act(o)
        r = 1.0 if a == best(o) else 0.0
        o2 = rng.normal(size=OBS_DIM).astype(np.float32)              # bandit: next state independent
        ag.push(o, a, r, o2, True); ag.learn()
        if step > 10_000:
            correct += (ag.act(o, greedy=True) == best(o)); total += 1
    print(f"DDQN self-test: greedy accuracy over last {total} steps = {correct/max(1,total):.2f} "
          f"(chance = {1/N_ACTIONS:.2f}); eps now {ag.epsilon():.3f}, buffer {len(ag.buf)}")

## scripts/train_marl.py

Config-driven training loop → experiments/<name>/.

"""Config-driven MARL training loop (SO2). Isolated: writes only to experiments/<name>/.
 
Each episode runs one corridor day under a MarlController (the shared DDQN acting + learning online).
Periodically evaluates the greedy policy and logs headway CV so you can watch it learn. This is the
harness you SWEEP — reward form/weights, ΔT, hyperparameters all live in Config (envs/marl_env.py);
nothing here is a committed decision.
 
Run from the repo root:
    python scripts/train_marl.py --episodes 6 --eval_every 3 --name smoke   # quick plumbing check
    python scripts/train_marl.py --episodes 800 --name gate                 # a real fail-fast gate
Scenario defaults to Stage A (D+T), holding-only — the gate condition. Writes metrics.csv + checkpoint.pt.
"""
import os, sys, time, csv, argparse, numpy as np
_here = os.path.dirname(os.path.abspath(__file__))
sys.path.insert(0, os.path.join(_here, "..", "envs"))
sys.path.insert(0, os.path.join(_here, "..", "agents"))
from corridor_sim import simulate
from marl_env import Config, MarlController, N_ACTIONS
from obs import OBS_DIM
from ddqn import DDQNAgent
 
 
def train(cfg, scenario=dict(T=True), eval_every=50, eval_seeds=5):
    exp = f"experiments/{cfg.name}"; os.makedirs(exp, exist_ok=True)
    cs = list(cfg.control_stops)
    agent = DDQNAgent(OBS_DIM, N_ACTIONS, lr=cfg.lr, gamma=cfg.gamma, buffer=cfg.buffer,
                      batch=cfg.batch, target_every=cfg.target_every, warmup=cfg.warmup,
                      eps_start=cfg.eps_start, eps_end=cfg.eps_end, eps_decay=cfg.eps_decay,
                      hidden=cfg.net, seed=cfg.seed)
    log = open(f"{exp}/metrics.csv", "w", newline=""); w = csv.writer(log)
    w.writerow(["episode", "train_return", "train_cv", "eval_cv", "epsilon"])
    t0 = time.time()
    print(f"[{cfg.name}] reward=({cfg.irr},{cfg.wait},{cfg.skip}) w={cfg.w} skip={cfg.skip_enabled} "
          f"stops={cs} scenario={scenario}", flush=True)
    for ep in range(cfg.episodes):
        ctrl = MarlController(agent, cfg, training=True)
        r = simulate(ctrl, seed=1000 + ep, control_stops=cs, **scenario)
        ctrl.finalize()
        eval_cv = ""
        if (ep + 1) % eval_every == 0:
            evs = [simulate(MarlController(agent, cfg, training=False), seed=90000 + s,
                            control_stops=cs, **scenario)["headway_cv"] for s in range(eval_seeds)]
            eval_cv = float(np.nanmean(evs))
            print(f"  ep {ep+1:4d}  ret {ctrl.ret:7.1f}  train_cv {r['headway_cv']:.3f}  "
                  f"eval_cv {eval_cv:.3f}  eps {agent.epsilon():.2f}  ({time.time()-t0:.0f}s)", flush=True)
        w.writerow([ep + 1, round(ctrl.ret, 2), round(r["headway_cv"], 4), eval_cv,
                    round(agent.epsilon(), 3)]); log.flush()
    agent.save(f"{exp}/checkpoint.pt"); log.close()
    print(f"[{cfg.name}] done in {time.time()-t0:.0f}s -> {exp}/ (checkpoint.pt, metrics.csv)", flush=True)
    return agent
 
 
if __name__ == "__main__":
    ap = argparse.ArgumentParser()
    ap.add_argument("--episodes", type=int, default=800)
    ap.add_argument("--eval_every", type=int, default=50)
    ap.add_argument("--eps_decay", type=int, default=30_000, help="exploration decay in steps (lower = exploit sooner)")
    ap.add_argument("--name", default="gate")
    a = ap.parse_args()
    train(Config(episodes=a.episodes, eps_decay=a.eps_decay, name=a.name), eval_every=a.eval_every)

## scripts/eval_marl.py

Evaluate a checkpoint as the fourth controller vs NC/FH/EH.

"""Evaluate a trained MARL checkpoint as the 4th controller vs NC/FH/EH (SO3 / the Dec-5 comparison).
 
Runs the greedy policy (and, by default, the three baselines) across the manuscript activation-matrix
cells at the SAME control stops and matched seeds, and reports headway CV / wait with bootstrap 95% CIs
plus the paired % change vs No-Control. This is the table the MARL result slots into.
 
    python scripts/eval_marl.py --ckpt experiments/gate/checkpoint.pt --N 20
"""
import os, sys, argparse, numpy as np
_here = os.path.dirname(os.path.abspath(__file__))
sys.path.insert(0, os.path.join(_here, "..", "envs"))
sys.path.insert(0, os.path.join(_here, "..", "agents"))
from corridor_sim import simulate, BASELINES, STOPS
from marl_env import Config, MarlController, N_ACTIONS
from obs import OBS_DIM
from ddqn import DDQNAgent
 
SCEN = [("Stage A (D+T)", dict(T=True)), ("Ablation S (D+T+S)", dict(T=True, S=True)),
        ("Ablation W (D+T+W)", dict(T=True, W=True)), ("Ablation B (D+T+B)", dict(T=True, B=True)),
        ("Stage B (D+T+S+W+B)", dict(T=True, S=True, W=True, B=True))]
 
 
def boot(x, n=5000, rng=np.random.default_rng(0)):
    x = np.asarray([v for v in x if np.isfinite(v)])
    if len(x) < 2: return (float("nan"), float("nan"), float("nan"))
    bs = [np.mean(rng.choice(x, len(x), True)) for _ in range(n)]
    return float(np.mean(x)), float(np.percentile(bs, 2.5)), float(np.percentile(bs, 97.5))
 
 
def pct(nc, o, n=5000, rng=np.random.default_rng(1)):
    nc, o = np.asarray(nc), np.asarray(o); m = np.isfinite(nc) & np.isfinite(o); nc, o = nc[m], o[m]
    if len(nc) < 2: return float("nan")
    return (o.mean() - nc.mean()) / nc.mean() * 100
 
 
def evaluate(ckpt, cfg, N=20, with_baselines=True):
    agent = DDQNAgent(OBS_DIM, N_ACTIONS, hidden=cfg.net, seed=cfg.seed); agent.load(ckpt)
    cs = list(cfg.control_stops)
    ctrls = (["NC", "FH", "EH"] if with_baselines else []) + ["MARL"]
    D = {}
    for name, kw in SCEN:
        for c in ctrls:
            cvs, wts = [], []
            for s in range(N):
                dec = MarlController(agent, cfg, training=False) if c == "MARL" else BASELINES[c]
                r = simulate(dec, seed=s, control_stops=cs, **kw)
                cvs.append(r["headway_cv"]); wts.append(r["wait_s"])
            D[(name, c)] = (cvs, wts)
        row = " | ".join(f"{c} {boot(D[(name,c)][0])[0]:.3f}" for c in ctrls)
        pv = "" if not with_baselines else "  vs NC: " + ", ".join(
            f"{c} {pct(D[(name,'NC')][0], D[(name,c)][0]):+.0f}%" for c in ctrls if c != "NC")
        print(f"{name:20s} | {row}{pv}", flush=True)
    return D
 
 
if __name__ == "__main__":
    ap = argparse.ArgumentParser()
    ap.add_argument("--ckpt", required=True)
    ap.add_argument("--N", type=int, default=20)
    ap.add_argument("--no-baselines", action="store_true")
    a = ap.parse_args()
    print(f"control stops: {[STOPS[i] for i in Config().control_stops]}  N={a.N}")
    evaluate(a.ckpt, Config(), N=a.N, with_baselines=not a.no_baselines)